Patrick LINDECKER (F6CTE) the 18th of july 2004

SPECIFICATIONS OF THE PSKAM10/31/50 MODES (MULTIPSK version 2.4)

GENERAL SPECIFICATIONS

Creator: Patrick Lindecker F6CTE (2002/2003)

Description :

Baud rate 	: 10 (PSKAM10) , 31.25 (PSKAM31), 50 (PSKAM50)

Speed 	: 6 wpm (PSKAM10), 20 wpm (PSKAM31), 31 wpm (PSKAM50),

Modulation 	: DBPSK

Reception mode: indifferent (LSB or USB)

Dictionary 	: 56    “ fixed length ” characters A..Z 0..9    <CR> <SPACE> . , : - = + ? $ ’ ( ) ! / @    %  with an error reset character and an idle character (displayed by ‘>’). Small characters are transformed in capital characters and special characters as “ é ”    are transformed in standard capital (in the example: “ E ”)

Shape of pulse 	: for PSKAM10 and PSKAM50: rectangular but the connection between 2 successive 		different bits is done by a decreasing sine then by a creasing sine (each of one during 		quarter of period)

for PSKAM31: raised cosine

Bandwidth 	: about 50 Hz (PSKAM10), 80 Hz (PSKAM31), 260 Hz (PSKAM50),

Demodulation 	: coherent,

Synchronization: automatic using the signal

Correction code: no

Convolution code: no

Interleaving	: no

Repetition 	: each character is repeated 5 positions later (so 0.8 s in PSKAM, 1,28 s in PSKAM31 or 4 s in PSKAM10)    between 2 transmissions of the same character) (example: A X B Y C A D B…). This diversity in time allows the correction of an erroneous character. Its detection is easy due to the particular content of a character (5 “ space ” and 3 “ mark ”).

Automatic Frequency Correction: from +/-3 Hz to +/- 10 Hz depending on signal level,

Drift tolerance 	: 60 Hz/mn for PSKAM31 and PSKAM50 and 40 Hz/mn for PSKAM10 (depending on signal level)

Lowest S/N for 2% errors: -11 dB for PSKAM50, -13 dB for PSKAM31 and -19 dB for PSKAM10 (for white noise in a 3 kHz bandwidth)

CHOICE OF THE COMMUNICATION MODE

The most simple is a two phases modulation in a biorthonormal form: 0° phase and 180° phase (inverse). It must be seen as BPSK (binary PSK) or binary PAM. It is also a very efficient mode to transmit in white noise with a low probability of symbol error.

To avoid any phase reference, it will be chosen a differential mode (what is detected is not the phase but its inversion or not: 0°--->180 ° or 180°--->0° for inversion and 0°--->0 ° or 180°--->180° for non-inversion). Unfortunately, the bit error rate is doubled.

The detection remains coherent (in fact “ quasi-coherent ” because the synchronization is not sure but built with the signal).

Note: it’s called “coherent” because a PLL is used to follow the phase. But the phase is known with an ambiguity of 180 °, so it’s not a real “coherent” detection.

The transmission may be done in LSB or USB. But to simplify, USB is recommended so when a frequency is proposed everybody speak of the same QRG.

SHAPE FOR PULSE

For PSKAM31

The type of filter used is the one used for PSK 31 (standard raised cosine). The envelope of the level is constant (except during transitions) so the average power is about 70 % of the maximum power (obtained for example with a “tune”).

For PSKAM10 and 50

The type of filter used for sending bits has been chosen after some experimentations.

I finally choose a special shape of pulse: rectangular but the connection between 2 successive different bits is done by a decreasing sine then by a creasing sine (each of one during a quarter of period). This allows a not too large bandwidth without interference between symbols. Except during transitions, the envelope of the level is constant so the average power is 85 % of the maximum power (obtained for example with a “tune”).

Example: 1011:   1    |   0   |  1    |   1

R-R-R-D-C-R-R-D-C-R-R-R-R-R-R-R

or C                          or D

R: rectangular (=1) of ¼ T. D: Sine decreasing of ¼ T (1 to 0)

C: Sine creasing of ¼ T (0 to 1)

BITS DISPOSITION AND DICTIONARY

For bits disposition, it’s inspired from AMTOR (fixed length). But to avoid the problem to shift between letters and figures (which drives to errors in packet), it has been chosen to create an original code of 8 bits.

Note: I have tried, first, a Varicode: it’s not a good solution...a lot of errors due to a complex interpretation.

This one is 56 characters long in place of 35 in AMTOR.

Each character contains 5 “0” and 3 “1”, which eliminates 256-56=200 invalid characters.

The reception of bits is done from left to right (“ 0 ” is a phase change and “ 1 ” a continuous phase as in PSK31).

With 8 bits length and 31.25 bauds, the speed in 19,53 wpm (31.25/8/2*60/6) which is the minimum reasonable.

'A': DEFINITION:='01100100';

'B': DEFINITION:='00100101';

'C': DEFINITION:='11000001';

'D': DEFINITION:='00001110';

'E': DEFINITION:='00100110';

'F': DEFINITION:='11000010';

'G': DEFINITION:='00001101';

‘H': DEFINITION:='00100011';

'I': DEFINITION:='11000100';

'J': DEFINITION:='00001011';

'K': DEFINITION:='01100010';

'L': DEFINITION:='11001000';

'M': DEFINITION:='00010110';

'N': DEFINITION:='01010010';

'O': DEFINITION:='10110000';

'P': DEFINITION:='10000110';

'Q': DEFINITION:='01010100';

'R': DEFINITION:='11100000';

'S': DEFINITION:='00010011';

'T': DEFINITION:='01011000';

'U': DEFINITION:='10100001';

'V': DEFINITION:='00011001';

'W': DEFINITION:='01001001';

'X': DEFINITION:='10100010';

'Y': DEFINITION:='10010100';

'Z': DEFINITION:='01001010';

'-': DEFINITION:='00011100';

'?': DEFINITION:='01001100';

':': DEFINITION:='10100100';

'$': DEFINITION:='10001010';

'3': DEFINITION:='01000101';

'!': DEFINITION:='10101000';

'@': DEFINITION:='00110001';

' ': DEFINITION:='01000110';

'8': DEFINITION:='11010000';

'%': DEFINITION:='00110010';

'(': DEFINITION:='01000011';

')': DEFINITION:='10010001';

'.': DEFINITION:='00110100';

',': DEFINITION:='01100001';

'9': DEFINITION:='10010010';

'0': DEFINITION:='00111000';

'1': DEFINITION:='01010001';

'4': DEFINITION:='00011010';

''': DEFINITION:='00101001';

'5': DEFINITION:='10000101';

'7': DEFINITION:='00010101';

'=': DEFINITION:='00101010';

'2': DEFINITION:='01101000';

'/': DEFINITION:='10001001';

'6': DEFINITION:='00101100';

'+': DEFINITION:='01110000';

CHR(13):DEFINITION:='10001100';

CHR(8):DEFINITION:='10000011';{Error reset key}

'“': DEFINITION:='00000111';{phasing 1...displayed as ‘”‘}

'”': DEFINITION:='10011000';{phasing 2...not displayed}

‘“‘: phasing signal 1 and ‘”’: phasing signal 2

CHR(8): back space (Error reset key)

CHR(13): <CR> + <LF> (Carriage return + Line feed)

The small letters keystroke at the editor are translated in capital letters.

IMPORTANT: in the same order, all the accentuated letters are translated in the equivalent non-accentuated letter. For example,    é and É become E.

Precisely, here is a part of the program (PASCAL) for reception of characters:

CARAC_RECU is the received character, REUSSITE the flag to specify if the

character is from PSKAM character and CARAC_A_AFFICHER is the character to display.

PROCEDURE VERIF_CARACTERE_ET_TRADUCTION_PSKAM(CARAC_RECU:CHAR;

VAR REUSSITE:BOOLEAN;VAR CARAC_A_AFFICHER:CHAR);

BEGIN

REUSSITE:=FALSE;

CARAC_A_AFFICHER:=CHR(0);

{a...z}

IF (CARAC_RECU>='a') AND (CARAC_RECU<='z') THEN

BEGIN

REUSSITE:=TRUE;

CARAC_A_AFFICHER:=CHR(ORD(CARAC_RECU)-32);

EXIT;

END;

IF (CARAC_RECU='Ç') OR (CARAC_RECU='ç') THEN CARAC_RECU:='C';

IF (CARAC_RECU='ü') OR (CARAC_RECU='û') OR (CARAC_RECU='ù') OR

(CARAC_RECU='Ü') OR (CARAC_RECU='ú')    THEN CARAC_RECU:='U';

IF (CARAC_RECU='é') OR (CARAC_RECU='ê') OR

(CARAC_RECU='æ') OR (CARAC_RECU='Æ') OR (CARAC_RECU='ë') OR

(CARAC_RECU='è') OR (CARAC_RECU='É')    THEN CARAC_RECU:='E';

IF (CARAC_RECU='â') OR (CARAC_RECU='ä') OR (CARAC_RECU='à') OR

(CARAC_RECU='å') OR (CARAC_RECU='Ä') OR (CARAC_RECU='Å')

OR (CARAC_RECU='á') THEN CARAC_RECU:='A';

IF (CARAC_RECU='ï') OR (CARAC_RECU='î') OR (CARAC_RECU='ì') OR

(CARAC_RECU='í') THEN CARAC_RECU:='I';

IF (CARAC_RECU='ô') OR (CARAC_RECU='ö') OR (CARAC_RECU='ò') OR

(CARAC_RECU='Ö') OR (CARAC_RECU='ó')    THEN CARAC_RECU:='O';

IF (CARAC_RECU='ÿ') THEN CARAC_RECU:='Y';

IF (CARAC_RECU='ø') OR (CARAC_RECU='Ø') THEN CARAC_RECU:='0';

IF (CARAC_RECU='ƒ') THEN CARAC_RECU:='F';

IF (CARAC_RECU='¿') THEN CARAC_RECU:='?';

IF (CARAC_RECU='ñ') OR (CARAC_RECU='Ñ') THEN CARAC_RECU:='N';

IF ((CARAC_RECU>='A') AND (CARAC_RECU<='Z')) OR

((CARAC_RECU>='0') AND (CARAC_RECU<='9')) OR

(CARAC_RECU='-') OR (CARAC_RECU='?') OR    (CARAC_RECU=':') OR

(CARAC_RECU='$') OR    (CARAC_RECU='!') OR (CARAC_RECU='@') OR

(CARAC_RECU=' ') OR    (CARAC_RECU='%') OR (CARAC_RECU='(') OR

(CARAC_RECU=')') OR    (CARAC_RECU='.') OR (CARAC_RECU=',') OR

(CARAC_RECU='''') OR (CARAC_RECU='=') OR    (CARAC_RECU='/') OR

(CARAC_RECU='+') OR (CARAC_RECU=CHR(13)) OR (CARAC_RECU=CHR(8))

OR (CARAC_RECU='“')    THEN

BEGIN

REUSSITE:=TRUE;

CARAC_A_AFFICHER:=CARAC_RECU;

END;

END;

The idle character '“' is transparent for the user. The Error reset key corresponds to the "Back Space" (CHR(8)).

# DESCRIPTION OF THE PROTOCOL AND TREATMENT

Preliminary: The protocol is very similar to the one used in AMTOR FEC (Mode B) in Collective Mode.

##### THE STATION SENDING PSKAM

- Emits each character twice: the first transmission (DX) of a specific character is followed by the transmission of four other characters, after which the retransmission (RX) of the first character takes place, allowing for time diversity reception at 0.8 s time space for PSKAM50, 1.28 s for PSKAM31 and 4 s for PSKAM10.

- Emits as a preamble to messages , alternately, twice, the phasing signal 1 and the phasing signal 2 whereby phasing signal 1 is transmitted in the DX, and phasing signal 2 in the RX position. After the pair of phasing signal 1/ phasing signal 2, it is emitted a Carriage Return (CHR(13)) to pass at the next line.

- Emits during the breaks between two messages in the same transmission the phasing signals 1 and the phasing signals 2 in the DX and the RX position, respectively.

##### THE STATION RECEIVING PSKAM

##### Check both characters (DX and RX):

##### * If they are identical and valid: it is printed the character,

##### * If they are identical and invalid: it is printed ‘¿’ (error)…the more probable is a phasing lost,

##### * If the DX character is valid, the one in RX position is invalid, it is printed the DX character,

##### * If the DX character is invalid, the one in RX position is valid, it is printed the RX character,

##### * If the RX and DX characters are valid but different (and different from the couple phasing signal 1 / phasing signal 2), a search of the most probable character based on the levels of reception after the base band demodulation must be done: it is printed the most probable valid character,

##### * If the RX and the DX characters are invalid but different, a search of a possible valid character based on the levels of reception after the base band demodulation must be done: after which, depending on result, it is printed either a valid character or an error ‘¿’.

Note 1: the question of the treatment of resynchronization according to the number of errors is not fixed, but the goal is, of course, not to lose characters.

Note 2 : to take into account QSB and QRM which may do lose synchronization during several seconds, the software must continue to translate (with the preceding synchronization) during 10 seconds in PSKAM10 (3 s in PSKAM50 and 4 s in PSKAM31) considering the loss of characters as transitional. After, if synchronization has not been recovered, it’s considered as lost and the translation stops.

##### PHASING

##### * On receipt of the sequence “ phasing signal 1 ” - “ phasing signal 2 ”    separated by 4 valid characters in which “  phasing signal 1 ” determines the DX and “  phasing signal 2 ” the RX position. As soon as the two phasing signals have been determined in good position, phasing is acquired. This type of phasing is used at the beginning of the transmission and during idle characters transmissions.

##### * If signal has been yet emitted and only normal characters are transmitted, the phasing

##### must be acquired by looking for a sequence of 6*8=48 bits filling the following conditions:

##### - each successive byte is in accordance to the format 5 bits to “ 0 ” and 3 bits to 	“ 1 ” ,

##### - the 8 last bits are identical to the 8 first bits.

##### END OF EMISSION

There is no specific “ end of emission signal ”. Normally, the ham who wants to finish transmits +K (invitation to transmit) or a VA or a SK (end of transmission).

But it is obvious that the last RX characters must be transmitted so, in the DX correspondent positions, a phasing signal 1 must be found.

For example, to transmit ABC, a pause (of one character duration) and then +K:

DX 		RX			DX is the first position

“		”			RX is the second position

“		”	 		“ is the phasing signal 1

CHR(13)	” 			” is the phasing signal 2

A		”

B		CHR(13)

C		A

“		B

+		C

K		”

“		+

“		K