YO YAGI OPTIMIZER

Copyright 1994 by Brian Beezley, K6STI

All Rights Reserved

YO Yagi Optimizer..............................1

Input Files....................................2

Output Files...................................2

Running YO.....................................3

YO Conventions.................................3

Frequencies....................................4

Ground and Stacking............................5

Pattern-Quality Measure........................5

Optimization Tradeoffs.........................6

Optimization Method............................7

Elements Menu..................................9

Undoing Changes...............................10

Matching Networks.............................11

Driven Elements...............................13

Two-Element Yagis.............................14

Element Tapering..............................15

Element-Mounting Brackets.....................16

Boom Compensation.............................17

Frequency Scaling and Taper Modification......18

Conductivity Losses...........................19

Gain Figure-of-Merit..........................20

Algorithm Limitations.........................21

Notepad.......................................22

Element-Current Profile.......................22

Frequency Graphs..............................23

Pattern Plots.................................23

Changing Screen Colors........................25

Printing The Screen...........................26

Options Menu..................................26

SET Commands..................................28

Yagi File Format..............................33

Index.........................................37

---- YO YAGI OPTIMIZER -----------------------------------------

The YO.EXE program analyzes and optimizes Yagi-Uda

antenna designs on IBM PC compatible computers.  YO analyzes

Yagis hundreds of times faster than general-purpose antenna-

modeling programs.  A fast Yagi model is implemented with high

accuracy for forward gain, front-to-back ratio, input impedance,

SWR, and radiation patterns.

The mathematical model used by YO is more complex than

the W2PV Yagi model, simpler than the MININEC model, and is more

accurate than either.  YO accounts for conductor losses.  Yagis

with dual driven-elements can be modeled.  Antennas may be

modeled in free space or over an infinite, perfectly conducting

groundplane.  Two identical Yagis stacked in the H-plane may be

modeled in free space.  Mutual-impedance interactions are

calculated in all cases.

YO includes an automatic control loop to simultaneously

optimize forward gain, radiation pattern, and SWR across a

frequency band.  This iterative procedure automatically adjusts

all element lengths and spacings to converge a design to an

optimal set of dimensions using performance criteria and

parameter tradeoffs which you specify.

YO computes element currents using a very fast algorithm

which has been calibrated to NEC, the high-accuracy Numerical

Electromagnetics Code from the Lawrence Livermore National

Laboratory.  YO and NEC are typically within 0.05 dB for forward

gain, within a dB or two for F/B, and within a couple ohms for

input impedance.  Radiation patterns produced by YO and NEC are

nearly identical for most designs.  For comparison purposes, a

MININEC calibration is also provided.  The MININEC algorithm is

less accurate than NEC but widely available.  Many existing Yagi

designs are based on MININEC or on similar method-of-moments

procedures.

The high speed of the YO analysis algorithm permits

rapid design optimization.  On modest computer systems with a

math coprocessor, tens of automatic design evaluations per

second are possible for small Yagis.  Even on slow computers,

designs with several elements may be optimized over a frequency

band in just a few minutes.  For complex antennas like long-boom

VHF/UHF Yagis, the optimizer may be run unattended to find the

best design automatically (while you sleep, if necessary).

Yagis with up to 50 elements can be modeled.

YO can optimize a Yagi at a spot-frequency or at the

low, middle, and high frequencies of a band of interest.  The

far-field radiation pattern at each frequency is sketched on

your graphics screen.  The patterns, as well as forward gain,

F/B, input impedance, and SWR, are continuously updated as the

Yagi dimensions are iterated.  This running display lets you

monitor the progress of the optimizer at a glance.

1

---- INPUT FILES -----------------------------------------------

YO reads the geometry of a Yagi from an input text file.

The file specifies analysis frequencies, contains an easy-to-

read table of Yagi dimensions, and permits a separate tapering

schedule for each element.  Element positions or spacings may be

given, whole- or half-element lengths are allowed, and you can

use U.S. units, metric units, wavelengths, or a mixture of all

three.  Yagi files always have the extension YAG.

Yagi files are ordinary text files composed of ASCII

characters.  You can create a Yagi file with any text editor or

wordprocessor.  If you use a wordprocessor, be sure to generate

a standard ASCII text file rather than a file in wordprocessing

format.  Files in wordprocessing format contain embedded control

codes which will confuse YO.

---- OUTPUT FILES ----------------------------------------------

To ensure that the current design is not inadvertently

lost, an output file named OUT.YAG is automatically written when

you exit YO, whenever you read a new input file, or when a

terminal error occurs.  You may recover the design by reading

OUT.YAG as an input file.

Use the Save command to write an output file

intentionally.  You may specify path and filename.  When you

specify the extension ANT, YO writes the file in AO/MN format.

When you specify YAG or no extension, the YO format is used.  If

you enter the name of a file which already exists, YO asks

permission before overwriting it.  See the SET Commands section

to disable this check.  Press just Enter to overwrite the last

file saved (SAVE.YAG is written if none has been saved).

Current design dimensions and file notes may be printed

by entering PRN or LPT for the output filename in the Save

command.

YO-format output files can be used later as input files

for further optimization.  Output files also are used to obtain

final Yagi dimensions, since YO does not display a complete set

of tapered-element dimensions on-screen.  Use a file-listing

utility, the DOS TYPE command, the DOS PRINT command, or your

text editor to examine YO output files.

For YO-format output files, element positions are

normalized so that the reflector position is always 0.  For AO/

MN-format output files, element positions are normalized so that

the center of the boom is always at X = 0.  Output files use the

units specified in the Options menu, not the units specified in

the input file.

2

---- RUNNING YO ------------------------------------------------

Before running YO, do the following, according to your

display type:

HGC:    The HERCULES graphics driver must be loaded.  To do

this, simply enter HERCULES at the DOS prompt.  This

needs to be done just once per boot.  HERCULES may be

loaded from your AUTOEXEC.BAT file for convenience.

CGA:    Load GRAFTABL and GRAPHICS once per boot (you may load

them from your AUTOEXEC.BAT file).  These are standard

DOS programs.  They provide some additional CGA fonts

and permit the CGA screens to be printed.

EGA/VGA:  No preliminaries are required.

The quickest way to start YO is to give the name of a

Yagi file on the command line, for example, YO NICE5.  If you

don't know which file you want, just type YO and Yagi files in

the current directory will be listed.

Select a file by moving the lightbar with the arrow

keys, PgUp, PgDn, Home, or End.  Press Enter to select the

highlighted file.  Alternatively, you may type a filename.  As

you type, the lightbar moves to the first filename which matches

the characters entered.  Press Enter whenever the desired file

is highlighted.

Select the last item ("Other") to enter a file or

directory name not listed.  If you enter a directory containing

Yagi files, YO lists them and you may select one.  See the SET

Commands section for information on organizing Yagi files into

subdirectories.

Instead of optimizing a Yagi file, you may display a

plot file generated previously.  To do this, start YO by typing

YO PLOT.  Plot files in the current directory will be listed and

you may select one.  Alternatively, you may specify the name of

a plot file on the command line, for example, YO PLOT NICE5.

This displays the plot immediately.

---- YO CONVENTIONS --------------------------------------------

YO turns off Caps Lock on startup.  Because some of the

YO commands are case-sensitive, it's best to leave this lock

off.

Press Esc to return to the main menu from a submenu.

You also may return by pressing the command key a second time

(but not from Save).  This is handy when making a quick check.

3

You may terminate data entry and return to the main menu

with one keystroke by pressing Esc instead of Enter.  With this

technique, when you return to the submenu the cursor is

positioned at the item last entered.  This makes it easy to

change a parameter repeatedly to experiment with its effect.

You may exit most submenus (not Save or Notes) by

pressing a valid command key.  The command will be executed

immediately, bypassing the main menu.  This is the quickest way

to go from submenu to submenu.  If a command key is pressed

during data entry, the data is accepted and the new command

executed.

Press Esc to hide the main menu to see the whole screen.

Press a key to bring it back.  If the key pressed is a command

key, the command will be executed immediately.

Use an arrow key with numerical input to enter data and

simultaneously move the cursor up, down, right, or left to

another item.  Use the spacebar to enter data without advancing

to another item.  This is useful with the Match and Bracket

menus to change a parameter value repeatedly to experiment with

its effect.

To reduce screen clutter, the numbers within the Yagi

patterns are not labeled.  They have the following meanings:

Frequency

Forward Gain

Front-to-Back Ratio

Input Impedance

Standing-Wave Ratio

Elevation Angle or Gain FOM

Elevation angle is displayed only for Yagis over ground.

Gain figure-of-merit is displayed only for single-Yagi, free-

space models.  Note that YO uses a generalized definition of F/B

(described later).

The iteration count in the lower-left corner is

incremented each time element currents are recalculated.  The

path and filename appear in the lower-right corner.

The symbol w is used for wavelength units.  Z stands for

impedance.  The notation 12.7-j15.4 means a resistance of 12.7

ohms in series with a reactance of -15.4 ohms.

---- FREQUENCIES -----------------------------------------------

YO models Yagi designs at a spot-frequency or over a

frequency band.  A single analysis frequency is specified for

spot-frequency designs, while low, middle, and high frequencies

are specified for designs covering a band.

4

You may change, add, or delete frequencies with the

Frequency menu.  You must use one or three frequencies, two are

not allowed.  To change to a spot-frequency design, enter 0 for

the low or high frequency.

Use the Options menu to select optimization at three

frequencies or at the middle frequency only.  It takes three

times longer to optimize at three frequencies.  When optimizing

at one frequency, it takes additional time to compute and

display the response at the other frequencies.  For maximum

speed, use the fewest number of frequencies.

A Yagi may be modeled at any frequency with YO.

Frequencies above 10 GHz are displayed with a leading %

character to indicate that frequency fields in output files will

overflow.  Special tweezers are available for those of you

working with Yagis above 10 GHz.

---- GROUND AND STACKING ---------------------------------------

Use the Ground menu to select the modeling environment.

When the antenna height and stacking distance are both zero, the

Yagi is modeled in free space.  Set the height to a nonzero

value to model the Yagi over a perfectly conducting groundplane.

The antenna is oriented for horizontal polarization when modeled

over ground.  The elevation angle is that at which forward gain

and the azimuth pattern are computed.  To model a stacked pair

of Yagis in free space, set the stacking distance to a nonzero

value.  YO takes mutual impedances into account when modeling

stacked Yagis or a single Yagi over ground.

---- PATTERN-QUALITY MEASURE -----------------------------------

YO uses a generalization of standard front-to-back power

ratio (F/B) as a measure of pattern quality.  Conventional F/B

is the ratio of forward power (at 0 degrees) to power radiated

in the opposite direction (at 180 degrees).  YO's generalized

F/B is the ratio of forward power to power radiated within a

specified region to the rear of the antenna.  Yagi designs

maximizing conventional F/B often have large backlobes at angles

other than 180 degrees.  Much better patterns result when Yagis

are optimized for F/B over an angular region.  The F/B region

begins at 180 degrees and extends forward to a limit set by you.

The limit is defined in 5-degree steps and may be set as far

forward as 5 degrees.  This wide range allows sidelobes to be

controlled as well as backlobes.

YO provides two methods for characterizing power within

the F/B region.  The first method simply averages power over the

region.  When F/B is optimized using this method, total power

within the F/B region is minimized.  This is particularly useful

with EME arrays to minimize total earth noise on receive.  The

second method uses an L4 norm.  This mathematical technique is

similar to that used to compute RMS power, but the largest

5

amplitudes are weighted much more heavily.  F/B computed using

the L4 norm is approximately the ratio of forward power to the

power of the peak backlobe within the F/B region.  When F/B is

optimized using peak power, backlobe amplitudes are minimized

while maintaining roughly equal amplitude.  This method is

particularly useful with HF Yagis since it minimizes worst-case

interference from signals arriving at random rear angles.  YO

samples the radiation pattern in 5-degree angular increments to

compute F/B.

You may define the F/B region with the Tradeoffs menu.

It may be set for 180 to 180 degrees (a single point) to obtain

conventional F/B.  Peak or average backlobe power may be

selected with the Options menu.  The F/B value depends on the

analysis plane selected, on the extent of the F/B region in that

plane, and on the power-computation method chosen.

For H-plane optimization of antennas over ground, the

F/B region begins at the rear horizon and extends upward to the

selected elevation angle.  This permits rear radiation to be

minimized over a range of elevation angles.

---- OPTIMIZATION TRADEOFFS ------------------------------------

Maximizing Yagi forward gain produces the strongest

transmit signal in the forward direction.  On receive, it yields

the best signal-to-noise ratio for omnidirectional noise.

However, it also results in a narrow main beam, a large

backlobe, multiple sidelobes, low input impedance, and small SWR

bandwidth.  These undesirable side-effects cannot be ameliorated

without giving up some forward gain.  Similarly, Yagis designed

for good patterns sacrifice gain, while those designed for wide

SWR bandwidth give up gain and pattern quality.

Because of the inherent tradeoffs in Yagi design, YO

permits a combination of forward gain, F/B, and SWR (or input

impedance) to be optimized.  The combination of parameters and

their relative importance is selected with the Tradeoffs menu.

The parameter tradeoffs are expressed in percent,

indicating relative importance.  For example, to maximize

forward gain regardless of the other parameters, set the gain

tradeoff to 100%.  To maximize gain but also take F/B into

account, set the F/B tradeoff to some nonzero value, say 50%.

This tells YO to weigh both forward gain and F/B when

optimizing, and in this case, to give them roughly equal

importance.  YO does this by forming a weighted sum of the

forward gain and F/B values and then maximizing the sum.

When optimization is allowed to run to completion, the

final design will be optimal in the following sense:  Any small

design change which yields a gain improvement of X dB will

necessarily reduce F/B by at least Y dB, where Y is related to X

by the ratio of the optimization summing weights.  In other

words, there will not be any free gain or free F/B to be had,

6

and to improve one parameter, the other must be sacrificed by a

factor proportional to its relative importance.

For three-frequency optimization, SWR may be minimized

while you're maximizing forward gain and F/B.  A function which

approximates worst-case SWR at the three frequencies is

minimized according to the SWR tradeoff value.  For single-

frequency optimization, impedance may be raised toward a

specified minimum value with the Impedance tradeoff.  (When

impedance is above the minimum, the impedance tradeoff is

effectively 0.)

YO scales the tradeoff percentages internally such that

equal 33% values usually yield a reasonable, balanced Yagi

design.  The 33% defaults form a convenient starting point for

custom optimization.  Vary the percentages according to the

importance you place on each design parameter.  The values

themselves are not important; it is their effect that counts.

For example, to emphasize F/B, simply increase the F/B tradeoff

value.  If too much gain is being sacrificed for SWR, raise the

gain tradeoff or lower the SWR tradeoff.  By adjusting the

tradeoff percentages, you can home-in on a design meeting your

requirements.  The design will be optimal in the sense that an

optimal tradeoff will have been made among the performance

parameters according to the importance assigned to each.

When optimizing at three frequencies, YO weighs the gain

and F/B values at individual frequencies according to the

Flatness parameter in the Tradeoffs menu.  When Flatness is 0%,

all frequencies are weighted equally and no attempt is made to

force a flat response.  This setting provides the highest

average gain and F/B across a band.  You can make the frequency

response more uniform by setting Flatness to a nonzero value.

However, keep in mind that forcing flat response always lowers

average gain and F/B somewhat.

Often SWR can be significantly reduced at very little

cost in forward gain or F/B.  After a design has been optimized

without regard to SWR, try introducing a small SWR tradeoff to

see what happens, even if the SWR is already acceptable.

Similarly, the impedance of single-frequency designs often can

be raised without significant loss of performance by

experimenting with a small impedance tradeoff.

During optimization, elements are not allowed to migrate

closer than .05 wavelength.  To change this limit, see the SET

Commands section.

---- OPTIMIZATION METHOD ---------------------------------------

YO uses a steepest-descent optimization technique.  This

simple hill-climbing method works as follows:

First, each element length and position is changed

individually by a very small amount (with all other antenna

7

dimensions unchanged) to calculate the sensitivity of the

objective to each variable.  The objective is the weighted

combination of gain, F/B, and impedance/SWR which you've defined

with the tradeoffs.  The set of element sensitivities is called

a gradient.  The gradient is an n-dimensional vector, where n is

the number of elements plus element positions.  The gradient

vector points in the direction which maximizes the objective.

After the gradient is calculated, an iteration step is taken

where all element lengths and positions are updated

simultaneously along the gradient vector, each in proportion to

its respective sensitivity, and all in proportion to a step

size.  If the step size is not too large, the update will yield

a new design with incrementally higher performance.

Because it takes a long time to calculate the gradient,

the modified algorithm keeps iterating lengths and positions

along the same gradient vector until no further improvement

occurs.  A new gradient is then calculated and the process

repeated.  The algorithm automatically adjusts iteration step-

size for quickest convergence.  The optimizer quits when the

step-size has been automatically reduced many times in

succession, indicating that further improvement is not possible

with small perturbations of antenna dimensions.

This optimization technique is quite stable and

efficient, locating optimal dimensions for an antenna without

wild-goose chases in a reasonable number of iterations.

However, the method does not guarantee that the global optimum

is found (nor does any other optimization technique, except for

exhaustive search over all possible Yagi geometries, which is

impractical).  The global optimum is the very best set of

antenna dimensions possible, where "best" is defined by the

tradeoffs.  The global optimum is missed when the algorithm

converges to a local optimum, where small movement in any

direction produces worse results.

Optimization using perturbation techniques is inherently

nearsighted.  If you're maximizing forward gain, for example,

and there's a small bump in the gain curve for a certain set of

element lengths and positions, the design may wind up on top of

the bump because the algorithm cannot see beyond the immediate

vicinity to the highest point of the overall curve.  If you

imagine trying to climb to the very top of a mountain range with

multiple peaks in dense fog, you can visualize the problem.

Fortunately, local optima are encountered infrequently

with Yagis.  They usually occur only when a design undergoes

radical change.  If you believe that your design has converged

to a local optimum, you can modify antenna dimensions manually

and continue automatic optimization in a different region of the

performance space.

To avoid terminating optimization prematurely, YO

continues to iterate even when very small performance

improvements are being made.  Sometimes this slowly inches a

design into a new region of the performance space where

8

significant improvement becomes possible.  However, whenever you

think further improvement is unlikely, you can interrupt the

optimizer by pressing a key.

Press Enter or the spacebar to interrupt the optimizer

with no further action.  If you press a command key, the command

will be executed immediately.  For example, press Esc to stop

the optimizer and hide the main menu to view the whole screen.

If you start the optimizer with upper-case Z and let it

run to completion, it beeps twice when it's done.

---- ELEMENTS MENU ---------------------------------------------

You can change element lengths and positions by hand

with the Elements menu.  This is a great way to develop a feel

for the way each element affects the performance of a design.

Use the Elements menu for experiments, to touch up optimization

results, or to create new starting configurations for the

automatic optimizer.

Note that for tapered elements, lengths of electrically

equivalent cylindrical elements are shown.  These are not

overall tapered-element lengths and should not be used for

antenna construction.  See Element Tapering for more

information.

You can add and delete elements using the Ins and Del

keys.  Press Del to delete the element at the cursor.  When you

delete an inner element, the remaining elements are respaced to

fill the gap.  Press Ins to add an element below the cursor.

When you add an inner element, elements are respaced to avoid

bunching.  When you add or delete an end element, elements are

not respaced and the boomlength changes.  New-element length is

set to the average length of adjacent elements (new last-

director length is set to 0.995 times old last-director length).

Use the X key to fix/free both element position and

length during automatic optimization.  The I key fixes/frees

either position or length.  The * key fixes/frees the positions

or lengths of all elements.  Use * then X or I to optimize just

a few elements.  Fixed element dimensions are highlighted by

color for EGA/VGA and by underlining for other displays.

(Reflector and last-director positions are not highlighted when

a longer boom is disallowed with the Options menu.  These

elements are still free to move closer together.  Similarly,

lengths are not highlighted when longer elements are

disallowed.)

Use the D key to switch between single and dual driven

elements.  When you switch to dual, both dual-drive options in

the Match menu are enabled to help optimize what was previously

a single-driven design.  Any switch frees all elements (except

for the position of a single driven element).

9

Use the ~ key to round element positions and lengths.

This is an easy way to test design sensitivity to dimension

tolerances.  Rounding also can be used to generate final

dimensions for construction.  See the next section to undo

rounding experiments.  (Since length rounding is applied to the

tip sections of tapered elements, it's normal for cylindrical-

equivalent lengths not to be multiples of the rounding value.)

Changing the position of the last director has a special

effect:  all element positions after the last fixed-position

element change to expand or shrink the design.  The whole beam

scales if no element positions are fixed (only the last-director

position changes if it's fixed).  This feature is very handy

when determining optimal boomlength.

Before accessing the Elements menu, you can select an

element from the main menu with the left/right arrow keys.  The

selected element blinks in the Yagi sketch.  When you press E,

the cursor will be positioned at the selected element.  This

feature makes it unnecessary to count elements.

---- UNDOING CHANGES -------------------------------------------

YO automatically saves the current design on a last-in,

first-out stack whenever elements change due to the Optimize,

Elements, Match, or Bracket commands.  The stack maintains a

history of design changes.  You can recall a previous design

with the Undo command.  Each press of lower-case u moves back

one design.  Press upper-case U to move all the way back to the

first design on the stack.  Use the Redo command to move

forward.  Each press of lower-case r moves forward one design,

while upper-case R moves all the way forward to the current

design.

Undo and Redo eliminate the need to periodically save

intermediate designs with the Save command.  They make it easy

to undo manual element changes (you don't have to remember the

old dimensions).  Undo and Redo provide a great way to compare

results before and after optimization (particularly when you use

the graph-comparison feature).

Undo and Redo have several limitations.  Only

cylindrical-equivalent element lengths, element positions,

number of elements, number of driven elements, dual-drive

phasing-line impedance, and transformer match frequency are

saved.  A design is saved when the optimizer terminates (if the

elements have changed), but intermediate designs are not saved

during optimization.  Scaling or retapering a design resets the

design history.  If you change a recalled design, history from

that point forward is lost.  YO saves the twenty most-recent

designs.

10

---- MATCHING NETWORKS -----------------------------------------

Use the M key to access the Match menu.  Use the / key

to select one of the following network types:

1.  Transformer Match

The transformer match consists of a broadband

transformer in series with a coil or capacitor.  The transformer

matches antenna input resistance to the feedline impedance,

while the series component tunes out antenna reactance.

The transformer match normally is used during

optimization.  It's always perfectly matched at one frequency.

YO automatically adjusts this frequency during optimization for

minimum SWR across a band.  (You can set the match frequency

manually to favor a certain portion of a band.  This disables

automatic adjustment.  To reenable it, set the match frequency

to 0.)

Although YO can minimize SWR while using another

matching network during optimization, it does not automatically

adjust network dimensions.  Because the transformer match is

always perfectly matched, it's convenient to use during

automatic optimization.

While the transformer match is seldom seen in practice,

it's useful as a reference for the inherent bandwidth properties

of the antenna.  The transformer is modeled as perfectly

broadband, and the series component has an insignificant effect

on SWR bandwidth.

2.  Hairpin Match

The hairpin match is an L-network using distributed

reactances.  The driven element is shortened from its resonant

length to raise the equivalent parallel input resistance to that

of the feedline.  The hairpin acts as a shunt inductance to

cancel the capacitive reactance of the shortened driven element.

Hairpin matches are commonly fed with coax through a balun.  YO

models the hairpin as a transmission line of parallel rods.

You specify rod diameter, rod length, and center-to-

center rod spacing.  You may also specify balun-lead length.

Enter the length of one lead from the axis of the balun to the

element-attachment point.  YO assumes the leads use #12 wire.

At HF, YO models the leads as fanning out from a spacing of 1.5"

at the balun to the hairpin-rod spacing at the element.  For

frequencies above 30 MHz, YO assumes the leads fan out from

0.15".  This lead spacing is typical of 3/8"-diameter coax

(presumably loaded with a few ferrite beads).

You may specify shunt capacitance between driven-element

halves.  This is particularly useful with Hy-Gain element

11

clamps.  Use 26.5 pF for 20-meter clamps and 9.5 pF for 15- and

10-meter clamps (tnx WB0DGF & W6QHS).

The Beta match is Hy-Gain's version of the hairpin

match.  It uses two rods which straddle the boom and a shorting

strap which connects all three together.  The presence of the

boom has a negligible effect on inductance, so the Beta match is

electrically equivalent to a conventional hairpin match.

NOTE:  When using a hairpin match, be sure to measure

driven-element half-length from the point where the feedline and

hairpin attach to the element, not from the centerline of the

boom.

3.  Gamma Match

The YO gamma-match model uses equations developed by

Harold Tolles, W7ITB, from Ham Radio Magazine, May 1973.  You

specify gamma-rod diameter, rod length, center-to-center spacing

from the driven element, and series capacitance.  You may also

specify lead length from the gamma rod to the coax connector

(#12 wire is assumed).  Only lead inductance is modeled (it's

really a transmission line with the boom as one conductor).

Specify lead length only when the lead does not run parallel to

the driven element.

For driven-element diameter, YO uses a weighted

logarithmic average of the diameters of the taper sections

spanned by the gamma rod (the diameters are weighted by taper-

section length).

4.  T Match

A T match is a balanced gamma match.  Rod length and

capacitance values apply for each side of the driven element.

Some T matches are fed with 50-ohm coax through a 4:1 half-wave

balun.  Enter a feed impedance of 200 ohms for these systems

(which may have negligible lead length).  You can also feed a T

match with wire leads from a 1:1 balun.  YO makes the same

assumptions for T-match balun leads as for hairpin-match balun

leads (the distance between rod ends is assumed to be half their

distance to the driven element).

To model a T match without capacitors, enter a high

capacitance value.  To model a folded dipole, extend the T-match

rod to the end of the element.

T-match and gamma-match rods increase the effective

diameter the inner part of a driven element.  Because YO does

not model this effect, you may need to compensate by increasing

driven-element length when constructing a Yagi.

12

A velocity factor of 0.975 is used for the transmission

lines formed by gamma rod and driven element, T rod and driven

element, hairpin rods, and balun leads.  Rod spacing is not

drawn to scale on the Yagi sketch, but rod length is.

Matching-network performance depends critically on a

number of parameters which are difficult to measure and to model

accurately.  Calculated and actual matches may differ due to

input-impedance modeling error, series- or shunt-capacitance

estimation error, lead-spacing variation, proximity of other

antennas or guy wires, etc.  When constructing a Yagi, always

make the matching network adjustable!

---- DRIVEN ELEMENTS -------------------------------------------

Driven-element length has very little effect on the

forward gain and pattern of Yagis with a single driven element.

This dimension normally is fixed during automatic optimization

(you might let YO optimize driven-element length when using a

hairpin match).  YO normally varies the length of both driven

elements during optimization of dual-driven Yagis.

YO can model Yagis using two driven elements

interconnected by a crossed phasing line.  This type of Yagi was

first popularized by KLM Electronics, Inc.  Dual-driven designs

sometimes are capable of lower SWR across a frequency band than

conventional designs.  Dual drive can be specified in the Yagi

file or with the Elements menu.

Dual-driven Yagis manufactured by KLM use balanced feed

at the forward driven element.  Nominal feed impedance is 200

ohms, and a 4:1 balun is used with 50-ohm coax.  Phasing-line

characteristic impedance varies with KLM antenna model and with

line assembly, but is in the range of 200 to 250 ohms for HF

Yagis.  YO uses default values of 230 ohms for phasing-line

impedance and 200 ohms for feed impedance.  You can specify

other values in the Yagi file or with the Tradeoffs menu.  A

velocity factor of 0.975 is used for the phasing line.

SWR characteristics of dual-driven Yagis are quite

sensitive to phasing-line characteristic impedance.  For KLM

construction, impedance varies with phasing-line tension.  Two

insulators determine line spacing at the center crossover point,

but the spacing elsewhere varies with line tension.  This

affects average line impedance.  Measurement of a 14-MHz KLM

Yagi with a time-domain reflectometer and a vector-impedance

meter yielded an average impedance of 215 ohms for a somewhat

slack phasing line and 240 ohms for a taut line.

Because dual-driven Yagis are fed directly, the usual

Match menu is replaced by two special options.  The first option

allows automatic optimization of phasing-line impedance.  The

second option maintains constant spacing between driven elements

while allowing their joint position to vary.  This lets you

optimize joint driven-element position without reconstructing an

13

existing phasing line.  The length of each driven element is

adjusted during optimization.

A good strategy for optimizing a dual-driven Yagi is to

first optimize the antenna using a nominal phasing-line

impedance of 230 ohms.  Once you have a good design, save it,

and then enable automatic optimization of phasing-line

impedance.  Reoptimize to see if significant improvement is

possible.  Try various starting impedance values.  Only SWR

characteristics are significantly affected by phasing-line

impedance, but dramatic improvement often is possible.

To realize optimum SWR characteristics, you should be

prepared to adjust the phasing-line impedance of a KLM antenna.

Even if the computer model looks good at 230 ohms, your

particular phasing line may exhibit a different impedance.  The

line impedance may be changed easily by varying the spacing

between phasing straps in the crossover region.  Simply replace

the original KLM spacers with larger or smaller ones of your own

design.  Experiments have shown that average line impedance can

be raised by 50 ohms by increasing the crossover spacing to a

few inches and lowered by 50 ohms by decreasing the spacing to a

fraction of an inch.  Fortunately, you do not need to measure

impedance.  Simply adjust phasing-line spacing until SWR

characteristics match those predicted by YO.  (If you don't want

to adjust phasing-line impedance, you can adjust driven-element

lengths to reduce SWR.)

NOTE:  As with a hairpin match, be sure to measure the

half-length of a dual driven-element from the point where the

transmission line attaches to the element, not from the

centerline of the boom.

When YO generates an AO/MN-format output file for a

dual-driven Yagi, it uses a current source at each driven

element rather than modeling the phasing line with wires.  Note

that source magnitudes and phases will not be strictly valid if

you change the AO analysis frequency.  Moreover, very short

taper sections representing shorting straps joining parasitic

element halves will not be modeled correctly by MN versions

earlier than 4.0.  When necessary, use the Scale menu to combine

such sections with adjacent taper sections. This problem may be

avoided by using MN 4.0 with automatic segment-length tapering

or by generating an output file with untapered elements.

---- TWO-ELEMENT YAGIS -----------------------------------------

YO expects every Yagi to have a reflector element.  Two-

element Yagis with a driven element and director can be modeled

by placing a dummy reflector a great distance away from the two

active elements.  This arrangement will satisfy YO while leaving

the response of the two-element array unaffected.

14

---- ELEMENT TAPERING ------------------------------------------

Fast Yagi-modeling algorithms represent each element as

a thin cylinder of constant diameter.  A tapering algorithm is

necessary to convert elements of tapered telescoping tubing to

untapered cylindrical equivalents.  For practical purposes,

cylindrical and tapered elements are equivalent when their self-

impedances are equal.  Tapered elements are physically longer

than their cylindrical equivalents.  Tapering must be accurately

modeled to realistically characterize HF Yagis.

YO contains three tapering algorithms.  The first is the

method developed by Dr. James L. Lawson, W2PV, from "Yagi

Antenna Design" published by the ARRL.  While this algorithm has

been widely used for a number of years, it doesn't fare too well

when combined with an accurate Yagi analysis algorithm.  The

W2PV method works reasonably well with MININEC, but only because

frequency biases of the two algorithms tend to cancel.  A

MININEC study of a number of tapered dipoles and their W2PV

cylindrical equivalents showed that element self-impedances were

not very close.  In addition, self-impedance values diverged

rapidly with frequency excursion from the central design

frequency.

YO's second tapering algorithm is designed to correct

these shortcomings.  This algorithm modifies the W2PV method to

yield cylindrical-equivalents with self-impedances much closer

to those of the tapered originals.  It also better predicts the

real-world performance of Yagis with heavily tapered elements.

In addition to providing better equivalent lengths, the

modified-W2PV algorithm also calculates cylindrical-equivalent

diameters more accurately.  Diameters are calculated such that

the self-impedance of untapered and tapered equivalents match

over a wide frequency range, not just at a single frequency.  In

essence, element Q is not affected by the taper conversion.  In

contrast, standard W2PV implementations set the cylindrical-

equivalent diameter to 7/8", to the mid-element taper diameter,

or to the average diameter of the tubing sections.  Such

equivalent diameters are approximately correct for many taper

schedules in common use.  However, to accurately model Yagi

performance over a frequency band, either the equivalent

diameters must be accurately represented or the taper conversion

must be recalculated at each analysis frequency.  Self-impedance

errors away from the central design frequency can give a

misleading indication of performance at the band edges.

The third tapering algorithm is by Dr. David B. Leeson,

W6QHS, from "Physical Design of Yagi Antennas" published by the

ARRL.  This algorithm gives results similar to those of the

modified-W2PV method.  The W6QHS algorithm has the advantage of

being based on fundamental principles rather than on an

empirical data fit.  It has the disadvantage of not having been

fine-tuned to the biases of the YO analysis kernel.

15

Use the Options menu to select tapering algorithm.  The

cylindrical-equivalent diameter used for the original W2PV

method is the average diameter of the taper sections, where the

diameter contribution of each section is weighted by section

length.  This method avoids problems introduced by very short

taper sections representing element-mounting brackets.  Twenty

half-element taper sections may be defined (seven are displayed

on-screen).

When YO outputs a tapered Yagi file, it reverses the

tapering process.  Tapered-element tip lengths are calculated

such that cylindrical-equivalent element lengths match those of

the current Yagi design.

---- ELEMENT-MOUNTING BRACKETS ---------------------------------

A conductive element-to-boom mounting bracket increases

the effective diameter of that part of an element in contact

with the bracket.  The amount of increase depends on the size

and shape of the bracket.  YO uses the equations of D. Jaggard

from the W6QHS Yagi book to model flat mounting plates, Hy-Gain

clamps, angle brackets, and channel brackets.

Use the B key to access the bracket menu.  Select

bracket type with the / key.  After you specify bracket

geometry, YO calculates the length and diameter of a new

innermost taper section.  The length of this section is the

half-length of the mounting bracket.  Its diameter is

electrically equivalent to the original element and mounting

bracket combined.

When you return to the main menu, all elements of the

current design are updated with the new taper section.  If the

Yagi uses mounting brackets of various sizes, use YO to

calculate equivalent taper sections and then use your text

editor to modify individual elements in the Yagi file.  (When

doing this, don't forget to shorten the length of original

innermost taper sections by the length of the newly added

sections.)

If the existing innermost taper section is very short

(less than 1/20th of the element half-length), YO assumes that

the section already represents a mounting bracket.  Mounting-

bracket computations will alter this section; a new section will

not be added.  If the innermost taper section is very short but

represents a real piece of tubing (such as for element

reinforcement), you should calculate the mounting-bracket

correction and then add the new taper section to the Yagi file

with your editor.  Otherwise, YO will modify the reinforcement

section.

If you enter zero for mounting-bracket width, length, or

thickness (so that YO lists "None" for the equivalent taper),

any very short innermost taper sections are removed from the

16

taper schedule and consolidated with adjacent sections when you

return to the main menu.

The length of a mounting bracket is its dimension along

the element.  Enter the total length (both sides of the boom).

Width is the dimension perpendicular to the element.  Height has

no meaning for flat plates and is automatically computed for Hy-

Gain clamps.  For angle or channel brackets, height refers to

the height of the vertical extrusion(s) above the bottom

surface.  YO assumes that the element is in contact with two

surfaces of an angle bracket and with one surface of a channel

bracket.

YO beeps and displays "Too Big" when the diameter of the

equivalent taper section is greater than .05 wavelength or its

length is greater than that of the innermost piece of tubing.

Reflector tapering is used to generate the numbers

displayed during mounting-bracket calculations.  When YO adds or

modifies taper sections, the correct taper schedule is used for

each element.

The element-mounting method always should be taken into

account when modeling Yagis.  Large mounting brackets can

significantly affect frequency response, moving a desired

response completely out of a band in extreme cases.  Small

brackets can upset F/B carefully optimized at a spot frequency.

However, mounting methods which use compact hardware no larger

than the element diameter generally require no correction.

---- BOOM COMPENSATION -----------------------------------------

When an element is close to or passes through a

conductive boom, its resonant frequency is raised.  YO does not

model this boom effect, but you may use the information given

here to compensate manually.  All correction values apply to

element half-lengths.  These corrections should be applied by

increasing calculated element lengths when constructing a Yagi.

Conversely, to model boom effects, you must shorten measured

element lengths.

For elements mounted on flat plates in contact with a

boom, W2PV found that element half-lengths should be increased

by 3% of the boom diameter.  He says that this small correction

diminishes rapidly as the element is spaced away from the boom,

even by a small distance.

For noninsulated through-the-boom mounting, measurements

by Guenter Hoch, DL6WU, have been curve-fitted by Ian White,

G3SEK, to yield the following boom-correction formula:

C = 12.5975B - 114.5B^2

C is the half-element correction factor as a fraction of

boom diameter B in wavelengths.  B^2 means B-squared.  For

17

example, a boom diameter of .01 wavelength requires half-

elements to be corrected by 11.5% of the boom diameter.  The

experimental data underlying this formula came from booms with

diameters smaller than .055 wavelength.  G3SEK indicates that

correction factors for insulated through-the-boom mounting are

close to 50% of C.

---- FREQUENCY SCALING AND TAPER MODIFICATION ------------------

YO can scale any Yagi design to a new frequency while

maintaining essentially the same performance characteristics.

This feature provides an easy way to replicate a Yagi design on

another band without designing an entirely new antenna.  The

taper schedule may be completely different at the new frequency.

YO will take everything into account and generate a new set of

dimensions with electrical performance very close to that of the

original design.

In addition to frequency scaling, element-tapering

modifications may be made to a Yagi without altering design

characteristics.  This is useful when experimenting with tubing

diameters to determine the effect on array bandwidth.  It's also

useful when duplicating an existing electrical design using new

hardware.  For example, you might use this feature when you come

across a bargain hamfest beam having good aluminum but unusual

tapering and unknown electrical design.  You could replicate

your favorite Yagi design onto this beam without designing a new

antenna from scratch.

Although scaling can virtually duplicate Yagi

performance at a new spot-frequency, the response will not be

exactly the same across a new frequency band unless the

diameters of the cylindrical-equivalent elements happen to scale

in exact proportion with frequency.  This is unlikely to occur,

since practical mechanical considerations or tubing material on

hand usually dictate element diameters.  Moreover, the two bands

are unlikely to have the same percentage bandwidth.  Therefore,

after scaling a design you should check performance across the

new band.  Usually the unmodified performance of scaled antennas

is entirely satisfactory.  If not, the new design can be used as

the starting point for a quick reoptimization.

Frequency scaling and taper modification in YO does not

rely on a simple polynomial approximation for element self-

impedance as does the W2PV scaling technique, nor does it

attempt to match element reactances.  Instead, the method-of-

moments algorithm used to compute element self-impedance during

normal analysis is invoked iteratively for each element while

the tip length is adjusted.  Adjustment is stopped when the

phase angle of the complex self-impedance has converged to that

of the original element (tnx WA2PHW).

The YO file format permits each element to have a

completely different taper schedule.  This is very handy when

building a Yagi using elements obtained from a variety of old

18

antennas.  However, to keep things simple, when new taper

diameters or lengths are entered during scaling, the new

dimensions are applied to all of the elements.  You may keep the

existing taper schedule (including element differences) when

just frequency-scaling an antenna.  This is handy when moving

the response of a Yagi around within a band.

To view the current taper schedule, select Scale from

the main menu.  The seven innermost taper sections are

displayed.  Dimensions listed are for the reflector.  The

notation [var] is listed for tip-section length which varies

among the elements.  To return to the main menu without making

any changes, press Esc.

To scale a Yagi, first enter a new center frequency.

Press just Enter to keep the current frequency.  Next, enter new

taper diameters, beginning with the innermost, separated by

spaces.  Press just Enter to keep the current set of taper

diameters (including any differences among the elements).

Finally, enter new taper lengths.  The length of the tip section

need not be entered.  YO will compute this length for each

element.  Press just Enter to keep the current set of taper

lengths (including any differences among the elements up to the

tip sections).

Append any dimension with ' " ft in m cm mm w to use a

more convenient measurement unit.  Don't leave a space between

the number and the symbol.

If you enter a taper diameter greater than .05

wavelength, YO prompts you to reenter the diameters.  If the new

taper schedule causes an element tip to acquire a negative

length, you must reenter the taper schedule.

The tapering algorithm (W2PV, modified W2PV, or W6QHS)

affects the results of taper modification and frequency scaling.

The algorithm you prefer should be in effect before beginning

any scaling or tapering operations.

---- CONDUCTIVITY LOSSES ---------------------------------------

YO can account for ohmic losses in Yagis due to finite-

conductivity element material.  This effect is usually small at

HF, but it can be important at VHF/UHF.  Conductivity losses are

recomputed whenever the taper schedule is changed.  To keep

things simple, losses are not recomputed for the small length

changes made automatically during optimization or for manual

changes made with the Elements menu.  When computing losses, YO

takes into account element-current distribution, the position,

length, and diameter of taper sections, and skin-effect depth,

which depends on frequency, material resistivity, and material

permeability.

Use the Options menu to enable/disable modeling of

conductor losses.  6061-T6 aluminum alloy is assumed for element

19

material unless one of the following metals is specified in the

Yagi file:

Silver           1.59E-08     Pure silver

Copper           1.7241E-08   100% IACS

Aluminum         2.655E-08    Pure aluminum

6063-T832        3.25E-08     Aluminum alloy

Brass            6.4E-08      Yellow brass (35% zinc)

Phosphor Bronze  1.1E-07      5% tin

Steel            7.2E-07      Stainless steel type 302

Note that 6061-T6 aluminum alloy has a volume

resistivity of 4.01E-08 ohm-m, 51% higher than that of pure

aluminum.  You may also enter resistivity or conductivity values

directly as follows:

Resistivity 5.7E-08 1.5          ohm-m, rel. perm.

Conductivity 53 1                IACS, rel. perm.

The second parameter is relative permeability.  This

parameter is optional and defaults to 1.  Permeabilities greater

than 1 may be entered for ferromagnetic element material like

iron or steel.  IACS stands for International Annealed Copper

Standard.  A material's IACS value is its conductivity expressed

as a percentage of that of annealed copper.  Pure copper is

103.06 IACS.

---- GAIN FIGURE-OF-MERIT --------------------------------------

YO performs an approximate calculation for the maximum

forward gain possible on the current boomlength.  For free-

space, single-Yagi models, the difference between the current

gain and this maximum is displayed at each frequency.  This

number provides a very handy gain figure-of-merit (FOM), showing

at a glance how close your design is to realizing maximum-

possible gain.

To calculate maximum-possible gain, YO uses a simple

formula derived from a linear-regression curve-fit done by Tom

Ring, WA2PHW, on an extensive collection of high-performance

Yagi designs:

G = 10Log(5.4075B + 4.25)

where G is gain in dBd and B is boomlength in wavelengths.

In addition to the WA2PHW gain formula, two others are

available.  The first, from Rainer Bertelsmeier, DJ9BV, computes

the gain of DL6WU-design long-boom Yagis.  VHF/UHF Yagis

20

designed by Guenter Hoch, DL6WU, have become something of a

standard for high-performance, state-of-the-art designs:

G = 7.773Log(B) + 9.28

The last gain formula is from Bill Myers, K1GQ, for HF

Yagis:

G = 3Ln(B) + 9.85

YO uses the WA2PHW formula by default because it gives

good results for any boom longer than 0.25 wavelength.

None of the formulas represent theoretical absolute-

maximum-gain designs.  This condition leads to extremely low

impedances and impractical antennas.  The formulas were derived

from gains of practical near-maximum-gain Yagis with real-world

losses and design compromises.  Because of this, some

exceptional designs have positive FOMs.  It's easy to change the

coefficients of the gain formula to suit more- or less-stringent

conditions, to better reflect your own experience, or to fit

theoretical calculations.  See the SET Commands section for

details.

The gain FOM is not intended as an overall quality

rating for Yagis.  It rates gain/boomlength efficiency only.

Most Yagi applications require good patterns and this constraint

always places a limit on forward gain.

---- ALGORITHM LIMITATIONS -------------------------------------

NEC calibration comes very close to actual antenna

performance at VHF/UHF as long as the element-mounting method is

properly taken into account.  MININEC calibration provides

results similar to those of NEC, but at a modeling frequency

anywhere from 0.25% to 2% higher, depending on element diameter.

NEC calibration is also very good at HF.  However,

additional error may arise from modeling element tapering and

mounting brackets, and from the proximity of guy wires, other

antennas, and structures.  These additional sources of error, as

well as smaller relative element diameters, tend to dilute the

NEC accuracy advantage over MININEC at HF.  Still, NEC

calibration is more accurate, and it's inherently safer.  Well-

designed NEC-calibrated Yagis stay clear of the high-frequency

rolloff region where Yagi directors rapidly approach resonance.

Yagis operated in this region exhibit marked dropoff in forward

gain, F/B, and input impedance.  MININEC's inherent frequency

offset can inadvertently bias designs into this region.

YO closely tracks NEC for element diameters up to .01

wavelength (0.27"-diameter elements at 432 MHz).  Yagis with

thicker elements may exhibit some frequency offset from

calculated designs, although YO gives reasonable agreement with

NEC for diameters up to about .04 wavelength.

21

YO models with input impedances of just a few ohms are

not likely to be accurate.  At these impedance levels, element

currents are very large and nearly cancel.  This condition

greatly magnifies small model inaccuracies.  Low-impedance Yagis

should be avoided for practical reasons as well.  Dimensions

become much more critical, skin-effect at VHF/UHF can result in

considerable loss, impedance matching is more difficult, and

bandwidth is severely restricted.

Input-impedance accuracy decreases for maximum-gain or

long-boom designs.  YO is typically a couple ohms high for

maximum-gain designs with impedances under 10 ohms.  YO is

typically a few ohms low for long-boom designs with impedances

in the 20- to 30-ohm region.

For very long-boom designs with element half-lengths

shorter than about 0.18 wavelength, YO may exhibit a frequency

offset of several percent (antenna characteristics occurring

lower in frequency than calculated).

---- NOTEPAD ---------------------------------------------------

You can enter, edit, display, and save design notes with

the Notes command.  The Yagi title is displayed at the top of

the notepad and may be edited.  The title and notes are saved in

all output files.

The notepad editor automatically wraps words within a

paragraph.  A blank line or a line with leading space begins a

new paragraph.  Use the arrow keys and Home, End, PgUp, and PgDn

to move the cursor.  Press Ctrl with the right or left arrows to

move the cursor to the beginning of the next or previous word.

Use Del or Backspace to delete characters.  Press Ctrl with

Enter to delete a word (when the cursor is between words, all

spaces but one are deleted).  Press F9 to delete a line.  See

the SET Commands section to redefine keys for the word-delete

and line-delete functions.  F10 (PrtSc for CGA) prints the

notepad screen.

---- ELEMENT-CURRENT PROFILE -----------------------------------

YO draws a profile of element-current magnitudes along

the Yagi sketch.  The curve may be turned on or off from the

Options menu.  Current is plotted linearly, where zero coincides

with the boom.  The profile is normalized so that curve height

remains constant from screen to screen.  Curve height may be

scaled with the up-arrow and down-arrow keys.  Scaling is useful

to bring out small detail or to position the curve for screen

printout.  The profile may be scaled during optimization without

interrupting the optimizer.

The element-current profile is fascinating to ponder for

its deeper significance, but it also has a practical use.  You

can use it to identify elements with questionable tuning,

22

particularly for long-boom Yagis.  Elements with abnormally high

or low current are easily spotted in the profile.

The current profile is computed at the middle frequency.

The profiles at the low and high frequencies can be quite

different and are interesting to compare.

---- FREQUENCY GRAPHS ------------------------------------------

YO graphs gain, F/B, SWR, and impedance curves versus

frequency.  Main-screen command keys are active while viewing

graphs.  For example, you can press M to abort graphing and

bring up the Match menu.  F5 changes screen colors, while F10

(PrtSc if CGA) prints the screen.

Graphs are scaled automatically.  41 frequency steps are

used (to change this number, see the SET Commands section).  The

F/B curve reflects current settings for F/B region and average/

peak backlobe power.  The impedance curve graphs the real part

of input impedance.  Spot-frequency designs are graphed over a

+/- 0.5% frequency range.

You can save the current graph by pressing F8 (the first

graph viewed is saved automatically).  Later, you can recall a

saved graph for comparison by pressing the spacebar.  The

spacebar then switches instantly between the two graphs.  This

is a great way to compare the overall performance of two

designs.  You can press F10 while viewing each graph to print

it.  The printed graphs can be compared easily since they use

the same scale.  Graph comparison is not available for CGA since

two hardware graphics buffers are required.

---- PATTERN PLOTS ---------------------------------------------

The sketches displayed during optimization use the

standard ARRL log-dB polar scale.  Half-scale is about 12 dB

down and quarter-scale is about 24 dB down.  Yagis plotted on

this scale produce familiar patterns which may be compared with

those in ARRL antenna publications and QST articles.  (See the

SET Commands section to use a linear-dB polar scale.)  Scale

markings are not shown to reduce screen clutter.  Yagi response

is computed every 5 degrees and the points are connected with

straight lines.

YO also contains a facility for plotting high-resolution

patterns.  This yields annotated polar and rectangular plots

with a variety of display options.  YO invokes this routine when

the P key is pressed and plots the center-frequency pattern in

1-degree resolution.

The P command causes a plot file to be generated and

displayed.  Patterns are saved in files so that you can review

and compare plots later.  A plot file uses the Yagi filename and

the extension PLT.  If you perform additional optimization and

23

press lower-case p, the plot file is overwritten with new

patterns and displayed.  If you press upper-case P, a new plot

file is generated.  Each new plot file has an incrementing digit

appended to its name.  Use of new filenames prevents previous

plot files from being overwritten, thus preserving plots for

later comparison.

Two plots may be overlayed.  In addition, switched plot

comparison is available on EGA, VGA, and HGC systems.  This may

be useful for comparing patterns with very detailed minor lobe

structure.  Plot comparison uses two hardware graphics buffers.

It can't be done on the CGA because it has just a single buffer.

You may view a plot from DOS by typing YO PLOT.  This

bypasses the rest of YO.  Alternatively, you may specify the

name of a plot file on the command line, for example, YO PLOT

NICE5.  This displays the plot immediately.

Free-space patterns are identified as "E-Plane" and "H-

Plane", while "Azimuth" and "Elevation" are used for over-ground

patterns (YO always orients Yagis for horizontal polarization

when modeling over ground).

YO normally begins by drawing a polar plot of the

E-plane or azimuth pattern using the log-dB scale (the linear-dB

scale is used when the POLAR environment variable has been set).

You may start in another mode by entering up to three option

letters after the filename as follows:

E       Elevation or H-plane pattern

R       Rectangular plot

L       Linear-dB scale

For example, YO PLOT DIPOLE ERL will cause YO to draw

the elevation or H-plane pattern in rectangular coordinates.  If

a polar plot is later displayed, it will use the linear-dB

scale.  Upper or lower case may be used.

The following keys are active in various modes during

plotting:

P           Plot using polar coordinates

R           Plot using rectangular coordinates

L           Switch the polar plot scale between log-dB and

linear-dB

X           Change X-axis cutoff for the rectangular plot

Y           Change Y-axis cutoff for the rectangular plot

A           Select another antenna and turn off plot

comparison and overlays

T           Change the trace thickness

O           Select another antenna and overlay its pattern

C           Select another antenna and compare its pattern

(not CGA)

N           Normalize compared plots to the same gain

scale; press again to revert to normal (not CGA)

Enter         Switch between azimuth and elevation (E/H) plots

24

Space         Switch between compared plots (not CGA)

PrtSc         Print screen (CGA)

F10          Print screen (EGA, VGA, HGC)

F5           Change colors (EGA, VGA)

Esc          Exit plotting

Other Keys      Activate the help panel, a context-sensitive,

abbreviated version of this list

Either of two radial scales may be selected for polar

plots.  The standard ARRL log-dB scale causes minor lobes to be

compressed toward the center of the pattern, emphasizing major

lobe shape.  This scale is widely used in amateur publications

and provides a convenient way to compare the patterns of

antennas you develop with those of existing designs.  On this

scale the center of the plot is minus-infinity dB (no signal),

but there is little plot area below -40 dB.

The other polar scale is a linear-dB scale, cut off at

-50 dB at the center of the plot.  This scale provides much more

area between -20 and -50 dB than the log-dB scale.  It's useful

for examining minor lobes which may be hard to see in a log-dB

plot.

The dots forming the radial lines (the ones which aren't

multiples of 30 degrees) are spaced every 2 dB.  The dots

forming the circles are spaced at multiples of 1 degree.  These

calibrations allow directivity values to be read from the plot

with good accuracy.

For free-space models, YO draws a 360-degree elevation

polar plot.  For models over ground, only the upper 180-degree

hemisphere is shown.

The polar plots are perfectly circular on monitors with

standard 4:3 aspect ratio.  Adjust your monitor's vertical-

height control to correct elliptical plots.

Rectangular (X-Y) plots may be selected with the R key.

This coordinate system can reveal minor-lobe detail even better

than the linear-dB polar plot, but overall pattern shape is not

quite as apparent.  The X-axis is degrees off boresight and the

Y-axis is antenna response in dB.  The Y-axis cutoff (lower

limit in dB) may be changed using the Y key.  This parameter is

always negative, but you may enter it without a minus sign for

convenience.  The Y-axis cutoff is constrained between -1 and

-100 dB.  The Y-axis is easiest to interpret when the cutoff

value is a multiple of 10 degrees.  The X-axis cutoff may be

changed with the X key.  The value entered is rounded to the

nearest multiple of 10 degrees within the range of 10 to 180

degrees.

---- CHANGING SCREEN COLORS ------------------------------------

Colors on EGA/VGA screens may be changed by pressing F5.

Use the left-arrow and right-arrow keys to select a screen item

25

for color change.  Then use the up-arrow and down-arrow keys to

cycle forward and backward among the 64 available colors.  Press

S to save color codes in the YO.INI file.  If this file is not

present when YO begins execution, a default color set is used.

You can easily return to default colors by deleting YO.INI.

Colors for the file-listing, high-resolution plot, and

frequency-graph screens also may be changed using F5.  Colors

for all screens are saved whenever those for one screen are

saved.

---- PRINTING THE SCREEN ---------------------------------------

YO can print EGA, VGA, and HGC screens to HP LaserJet-

compatible laser printers, to HP DeskJet printers, and to Epson-

compatible dot-matrix printers.  Press F10 to print.  To print

CGA screens or to use another type of printer, load the DOS

GRAPHICS program before starting YO.  Press PrtSc to print.  See

the SET Commands section for details on F10 printing.  See your

DOS manual for information on GRAPHICS.

The PSPLOT utility generates gorgeous plots on

PostScript printers from YO plot files.  PSPLOT can overlay

patterns and can do multiple plots per page.  PSPLOT is

available for $25 from Paul Terwilliger, NX1H, 11 North Shore

Rd., Chester, NH 03036, (603) 887-4346.

---- OPTIONS MENU ----------------------------------------------

The Options menu lets you control some additional

aspects of YO.

The E-plane pattern is normally optimized for

horizontally polarized Yagis since it determines azimuth

directivity.  Select the H-plane to optimize the azimuth pattern

of vertically polarized Yagis.  E-plane patterns for Yagis in

free space always have a null at 90 degrees from the pattern of

individual dipole elements.  Neither E-plane patterns of Yagis

over ground nor H-plane patterns have this null.  For these

patterns, a dot is placed at the plot origin for reference.

You can enable/disable modeling of conductor losses to

observe the effect on forward gain.

You can keep element lengths from increasing during

optimization.  This is very handy when optimizing an existing

antenna with nonadjustable element lengths and you don't want to

fabricate new elements.  This option lets you simply trim

existing elements for best performance.

When you disallow a longer boom, YO monitors the

position of the reflector and last director during optimization.

These elements will be allowed to migrate closer than their

initial distance but not farther apart.  The boom-length limit

26

is updated whenever you move these elements manually with the

Elements menu or you use Undo or Redo.

For experimental purposes, you can force director

lengths to decrease monotonically along the boom (length of

director n+1 less than or equal to length of director n).  This

feature has been included to demonstrate that this condition is

not necessary for optimal designs.

Select optimization at the middle frequency to optimize

a design at a spot-frequency while keeping an eye on response at

the band edges.  This method will not yield a Yagi with good

characteristics across a band, as will optimization at all three

frequencies (which takes three times as long).

Average or peak backlobe power may be selected for F/B

computation.  The F/B value changes whenever a different

computation method is selected.

MININEC calibration is provided for comparison purposes.

It's identical to that used in YO 3.0.

The element-current profile can be disabled to reduce

screen clutter.

When you select Save Options, a file named YO.INI is

written to the current directory.  Current settings for all

parameters in the Options and Tradeoffs menus and color codes

for all screens are saved in YO.INI.  Whenever YO begins

execution, it checks for this file.  If it exists, YO restores

all parameters from YO.INI.  Use Save Options to customize YO's

default settings.  (The Save Colors command in the color menu

does the same thing.)

YO can generate output files with tapered or with

electrically equivalent untapered elements.  Untapered AO/MN-

format output files may be used to compare the relative accuracy

of the AO and YO analysis kernels.  Tapered AO files may be used

to compare overall results of AO and YO, including effects of

the YO tapering algorithm.  Use caution when interpreting

results for tapered files with MN versions earlier than 4.0.

Prior MN versions can give erroneous results when very short

taper sections are present (such as those representing a few

inches of tubing swage).

When the tapering algorithm is changed, the current

design (internally modeled with untapered cylindrical elements)

is first converted to an equivalent tapered design with the

existing tapering algorithm, and then converted back to a

cylindrical-equivalent with the new tapering algorithm.

YO automatically converts dimensions in Yagi files to

the units specified in the Options menu.  Output files also use

these units.

27

---- SET COMMANDS ----------------------------------------------

DOS provides a convenient way to specify configuration

information to programs.  SET commands place information in the

DOS environment in memory where it can be retrieved later by a

program.  If you put SET commands in your AUTOEXEC.BAT file,

they will be executed automatically every time your computer

boots.  You may see what is in the environment by typing the

following:

SET

You may remove any parameter P from the environment by

typing:

SET P=

It's possible to run out of environment space.  To

increase its size to 512 bytes from the default of 256, put a

line similar to the following in your CONFIG.SYS file:

SHELL=\COMMAND.COM /E:512 /P

There are a number of SET parameters recognized by YO.

Don't put a space before or after the equal sign in SET

commands.

1. Subdirectories

Once you accumulate many Yagi and plot files, it's nice

to organize them into subdirectories.  You might use the current

directory for experiments, saving optimized Yagi files and plots

elsewhere.  You can tell YO to automatically reference certain

subdirectories with SET commands.  The subdirectories are used

only for reading files; YO always writes OUT.YAG and plot files

to the current directory.

Define subdirectories like this:

SET YAG=WIREBEAM VHF KLM .

SET PLT=. HFPLOTS VHFPLOTS WIREBEAM

The dot represents the current directory.  It may appear

anywhere (or nowhere) in a list.  You may define as many

subdirectories as you like.  Usually it's convenient to use the

same set of subdirectories for both Yagi and plot files.  You

can organize subdirectories by Yagi type, frequency band,

designer, etc.

Without SET commands, the subdirectory lists consist of

just the current directory.

YO (YO PLOT) begins by listing files in the first

subdirectory on the YAG (PLT) list.  Subdirectories are listed

after files.  Select one to list its files.

28

You may specify a file from one of the subdirectories on

the command line without typing the path.  YO searches the

subdirectories for the file in the order given in the

subdirectory list.

You may specify any subdirectory (not necessarily from

the list) on the command line and YO (YO PLOT) will list its

Yagi (plot) files.

2. Screen-Update Rate

During optimization, YO normally updates the screen no

more frequently than once per second.  If your computer uses a

math coprocessor and an AT-bus graphics card, updating the

screen can take a long time compared with the time required to

compute a design iteration.  All antenna calculation must be

halted while the screen is updated.  Frequent updates can slow

optimization considerably when the update rate is high.

However, the optimal screen-update interval for your

particular computer system may not be 1 second.  The best

interval depends on the relative speed of your CPU and graphics

card, on their absolute speed, and on your own personal

preference for screen activity versus optimization rate.

Screen-update overhead can be judged by looking for pauses in

the iteration count in the lower-left screen corner.  You may

change the screen-update interval with the following SET

command:

SET UPDATE=Time Interval

Enter the time interval in seconds (fractional values

like .5 may be used).  Set the time interval to 0 to display

every design iteration.  This lets you see all steps in the

evolution of a design and can be fascinating to watch.  However,

displaying every iteration on a computer with a fast CPU and

slow graphics can easily halve the optimization rate for small

Yagis.

3. Screen Bounce

When switched from graphics to text mode, screens of

some monitors bounce or break up during resynchronization.  YO

can momentarily blank the screen while things settle.  To set

this up, do this:

SET BOUNCE=Delay

Delay is in seconds.  Try 0.1 for EGA/VGA and 0.7 for

HGC monitors.

29

4. Reference dB

To display gain figures in dBi rather than dBd, do the

following:

SET DB=dBi

Gain in dBd is referenced to the peak gain of a half-

wave dipole in free space.  Gain in dBi is referenced to an

isotropic antenna in free space.  An isotropic antenna radiates

equally in all directions.  A dipole has 2.15 dB gain over an

isotropic antenna, so YO converts from one gain reference to the

other by adding or subtracting 2.15 dB.

5. Plot Normalization

YO normally scales the low-, middle-, and high-frequency

patterns independently.  This gives each plot the same amplitude

in the forward direction, making it easy to compare patterns.

However, gain differences are not apparent.  To show these, you

can use normalized plots where all patterns are drawn to the

same scale.  To set this up, do the following:

SET NORM=ON

6. Plot Resolution

YO normally generates high-resolution plots with

1-degree resolution.  This provides the best-looking plots, but

the plot files take somewhat longer to generate and to plot, and

they're twice as large.  To generate 2-degree plots, do the

following:

SET RES=2

7. Printer Type

The following SET commands apply only when using F10 to

print EGA, VGA, and HGC graphics screens.

To print normal-size plots on an Epson-compatible, 9-pin

dot-matrix printer, no SET command is required.

30

On some 9-pin printers, YO can generate an alternate,

larger plot for EGA/VGA screens.  To try it with your printer,

do the following:

SET PINS=9ALT

For a 24-pin dot-matrix printer, use:

SET PINS=24

To use an HP LaserJet or DeskJet printer, do this:

SET PINS=HPLJ

To try a landscape plot (not available on all printers),

use:

SET PINS=HPLJ L

For a bigger landscape plot, use:

SET PINS=HPLJ LB

8. Printer Port

Output goes to LPT1 by default.  For LPT2 or LPT3, use

one of the following:

SET LPT=LPT2

SET LPT=LPT3

9. Error Beep

YO beeps the speaker whenever an error occurs which

requires your attention.  To disable the beep, do the following:

SET BEEP=OFF

10. Notepad Editor Keys

The keys used by the notepad editor for word-delete and

line-delete may be redefined using WDEL and LDEL.  Use ^ to

indicate the Ctrl key and ~ to indicate the Alt key in

combination with a letter key.  ^Enter and ^Bksp may be used

also.  Use F1 through F10 for function keys.  For example:

SET LDEL=^Y

SET WDEL=~W

SET LDEL=F3

SET WDEL=^Bksp

31

11. Number of Frequency Steps for Graphs

If you have a slow computer, you may want to generate

frequency graphs faster.  If you have a fast computer, you might

like higher graph resolution.  The number of frequency steps

used to generate the graphs may be changed from the default

value of 41 with the following:

SET NFREQ=Number

12. Minimum Element Spacing

During optimization, elements are not allowed to migrate

closer than .05 wavelength.  HF elements spaced closer than this

limit may move enough in the wind to affect antenna performance.

Closer spacing may also result in increased modeling error.  You

may change the spacing limit with the following:

SET MINSP=Minimum Spacing

YO may be unable to optimize designs with elements

already spaced closer than the limit.  YO first respaces the

elements to satisfy the limit, but if this decreases

performance, the optimizer will quit without having made any

changes.  To overcome this snag, respace the elements by hand

and restart the optimizer.  Whenever the optimizer balks on an

obviously poor close-spaced design, this quirk probably is

responsible.

13. Linear-dB Plots

YO normally displays pattern sketches and high-

resolution plots using a log-dB polar scale.  You may specify a

linear-dB scale with:

SET POLAR=LINEAR

The linear-dB scale cuts off at -50 dB at the polar

center.

14. Plot-Trace Thickness

To default to thick plot traces, do this:

SET TRACE=THICK

15. Output-File Overwrite Check

To disable the check for overwriting an existing Yagi

file, do this:

SET FCHECK=NO

32

16. Gain Figure-of-Merit

To change the coefficients used in the WA2PHW gain

formula, do the following:

SET FOM=WA2PHW a b

where G = 10Log(aB + b).  G is gain in dBd and B is boomlength

in wavelengths.  If a and b are not specified, default values

are used.  Default values for the WA2PHW formula are a = 5.4075

and b = 4.25.

To select the DL6WU gain formula, use:

SET FOM=DL6WU a b

where G = aLog(B) + b.  Default coefficients are a = 7.773 and b

= 9.28.

To use the K1GQ gain formula, enter:

SET FOM=K1GQ a b

where G = aLn(B) + b.  Default values are a = 3 and b = 9.85.

When experimenting with the coefficients, note that a

controls the rate at which gain increases with boomlength, while

b affects the gain at all boomlengths.

When there is no FOM SET command, the WA2PHW gain

formula is used with default coefficients.  The FOM display can

be disabled entirely by setting FOM to anything other than

WA2PHW, DL6WU, or K1GQ.  For example:

SET FOM=OFF

---- YAGI FILE FORMAT ------------------------------------------

Here are some examples of Yagi files which illustrate

the file format.  Notes are kept at the end of a file after the

numerical information.  Elements must be specified in order,

beginning with the reflector.  One-half of each element is

normally defined (boom center to element tip).  However, for

untapered elements you may enter whole-element length (tip to

tip).

K7HYR's Max Gain Yagi

24.890  24.940  24.990  24.960  MHz

4 elements, inches

1.617     1.250     1.125     0.875

0.000     2.938    15.062    66.000    33.305

124.000     2.938    15.062    66.000    28.248

248.000     2.938    15.062    66.000    26.815

372.000     2.938    15.062    66.000    28.313

33

The first line is a descriptive title for the antenna.

YO truncates titles longer than 52 characters.

The second line specifies frequencies.  One or three

analysis frequencies may be given.  When three frequencies are

specified, you may also specify the transformer match frequency

(24.960 MHz in this example).  If not specified, YO automatic

adjusts the match frequency to minimize peak SWR after reading

the file.  You may use KHz, MHz, or GHz.  MHz is assumed if no

frequency units are specified.

Optional lines for stacking distance, antenna height,

dual drive, and element material may be added in any order

between the title and frequency lines.  These optional lines are

described later.

The third line gives the number of elements and the

dimension units.  You may specify up to 50 elements.  The comma

is optional.  You may use feet, inches, meters, centimeters,

millimeters, or wavelengths (only the first two letters of these

keywords are examined, so you can use metres, etc.).

Alternatively, you may use the symbols ' " ft in m cm mm w.

The next line lists taper diameters, starting with those

closest to the boom.  If you use tabs to separate the numbers,

including an initial tab as illustrated, the taper diameters

will line up with the taper lengths which follow.

The remaining lines list the element position followed

by the length of each taper section, beginning with the section

closest to the boom.  The reflector position need not be zero.

To use element spacings instead of positions, see the Cushcraft

12-4CD example below.

The maximum number of taper sections allowed is 20.

Taper lines may be as long as required to accomodate all the

sections.

You may use any combination of spaces, commas, and tabs

as separators.  Upper or lower case letters may be used for

keywords.

The example above uses the same taper schedule for all

elements.  Here is a Yagi which uses two different schedules.

Three taper diameter lines are used, the latest remaining in

effect until another is encountered.  This file also illustrates

how to model a Yagi over ground.  The height value uses the

dimension units specified for the rest of the Yagi, unless

overridden as in this example:

N2FB 6-el 20m, NCJ Jan 1986

Height 70'

14.000   14.174   14.350

6 elements, inches

1.250     1.125     1.000

34

0.000      18.000    54.000   138.970

1.000

86.380    196.578

1.250     1.125     1.000

164.090    18.000    54.000   126.640

321.590    18.000    54.000   121.950

464.450    18.000    54.000   121.900

643.990    18.000    54.000   113.520

YO distinguishes between diameter lines and length lines

as follows:  If the sum of the numbers (excluding the first) on

a line is < .15 wavelength, the line is a diameter line.

Otherwise, it's a length line.  Since taper lengths >= .3

wavelength are assumed to be whole-element lengths (YO divides

them by two), half-element lengths must be between 0.15 and 0.3

wavelength.

You may include taper lengths of zero for convenience.

The following taper schedule is equivalent to that above, but is

easier to read.  This file also illustrates how to specify a

stacked pair of Yagis in free space.  In this file the stacking

distance of 50 feet is expressed in inches, using the units

specified for the rest of the antenna:

N2FB 6-el 20m, NCJ Jan 1986

Stacked 600

14.000   14.174   14.350

6 elements, inches

1.250     1.125     1.000

0.000    18.000    54.000   138.970

86.380     0         0       196.578

164.090    18.000    54.000   126.640

321.590    18.000    54.000   121.950

464.450    18.000    54.000   121.900

643.990    18.000    54.000   113.520

If the last taper section for an element has zero length

(because the tubing size is used only for other elements) you

may omit the trailing zero altogether.  The following Yagi has a

0.5-inch diameter tip for the reflector only.  This file also

illustrates how element spacings may be specified instead of

positions.  The keyword "Spacing" may use upper or lower case

and may be singular or plural.  The spacing value for the

reflector has no meaning and can be any number, but must be

present.  This file also specifies copper elements.

Cushcraft 12-4CD

Copper

24.94 MHz

4 elements, inches

Spacing  .875    .75     .625    .5

0         25      45      45      6

35

67.625    25      45      45

47.375    25      45      42.5

95        25      29      44

For dual driven elements, insert a line beginning with

"DUAL" or "KLM" between the title and frequency lines.  To

change the default 230-ohm phasing-line impedance, add a value

to this line.  To change the default 200-ohm feedline impedance,

add a second value.  This line specifies 250-ohm phasing and

300-ohm feed:

Dual 250 300

Any of the Yagi dimensions may have a symbol appended to

use a different length unit than that specified on the number-

of-elements line.  This feature allows you to use feet for

length and inches for diameter, for example.  Don't leave a

space between the number and the symbol.  Here are the symbols:

' " ft in m cm mm w

36

INDEX

% character  5

' " ft in m cm mm w  19, 34, 36

* key  9

/ key  11, 16

180 to 180 degrees  6

200 ohms  12, 13

230 ohms  13

24-pin dot-matrix printer  31

4:1 balun  13

6061-T6  19

6063-T832  20

9-pin dot-matrix printer  30

^  31

~  10, 31

Above 10 GHz  5

Above 30 MHz  11

Absolute-maximum-gain designs  21

Accuracy  1, 21, 27

Add and delete elements  9

Algorithm  8, 15, 18

Algorithm Limitations  21

Allow a longer boom  26

Alt key  31

Aluminum  20

Always make the matching network adjustable!  13

Analysis frequencies  34

Analysis plane  6

Angle brackets  16

Angular region  5

Annealed copper  20

ANT  2

Antenna height  5, 34

AO  14

AO/MN  2

AO/MN-format output file  2, 14, 27

ARRL  15, 25

ARRL log-dB polar scale  23

Arrow key  3, 4, 10, 22

ASCII  2

Aspect ratio  25

AUTOEXEC.BAT  3, 28

Automatic optimization of phasing-line impedance  13, 14

Automatic segment-length tapering  14

Average backlobe power  6

Average line impedance  14

Azimuth  24

Azimuth directivity  26

B key  16

Backlobe amplitudes  6

Backlobe power  23, 27

Balun  11, 12

37

Balun leads  12

Band edges  15, 27

Bandwidth  11, 18, 22

Beep  9, 17, 31

Bertelsmeier  20

Best set of antenna dimensions  8

Beta match  12

Bias  15, 21

Blink  10

Boom  12

Boom Compensation  17

Boom diameter  17

Boom effect  17

Boom-correction formula  17

Boom-length limit  26

Boomlength  9, 20, 33

Bounce  29

Bracket  4, 16

Bracket menu  16

Brass  20

Broadband transformer  11

Bypassing the main menu  4

Capacitance  12

Caps Lock  3

Case-sensitive  3

CGA  3, 22, 23, 24, 26

CGA fonts  3

Change a parameter repeatedly  4

Change to a spot-frequency design  5

Channel brackets  16

Characterizing power  5

Clamps  12, 17

Close-spaced design  32

Coefficients  21

Color codes  27

Colors  23, 25

Command key  3, 4, 9

Command line  3, 24, 29

COMMAND.COM  28

Compare patterns  30

Complex self-impedance  18

Conductive boom  17

Conductivity  20

Conductivity Losses  19

Constant spacing between driven elements  13

Constructing a Yagi  13, 17

Conventional F/B  5

Conventions  3

Converge  18

Convergence  8

Copper  20

Correction factor  17

Correction values  17

Count elements  10

Crossover spacing  14

38

Ctrl  22

Ctrl key  31

Current  22

Current design  10, 27

Current directory  3, 27, 28

Current profile  22

Current source  14

Cursor  4, 9, 10, 22

Cylindrical-equivalent lengths  10

D key  9

Data entry  4

DBd  30

DBi  30

Deeper significance  22

Default  7, 13, 20, 21, 28, 31, 32, 33, 36

Default coefficients  33

Default colors  26

Default settings  27

Del  9

Delete a line  22

Delete a word  22

Delete characters  22

Deleting YO.INI  26

Design iteration  29

Design notes  22

Design sensitivity  10

Diameter lines  35

Dimension tolerances  10

Dimension units  34

Dimensions  2

Dimensions in Yagi files  27

Dipole  30

Directory  3, 28

Disable the beep  31

Disables automatic adjustment  11

DJ9BV  20

DL6WU  17, 20, 33

DOS  2, 3, 24, 26

DOS environment  28

Dot  25, 26, 28

Down-arrow  22

Driven element  11, 13

Driven Elements  13

Driven-element diameter  12

Driven-element length  13

DUAL  36

Dual drive  34

Dual driven elements  9, 36

Dual-driven Yagis  13

Dummy reflector  14

Duplicating an existing electrical design  18

E-Plane  24, 26

Earth noise  5

Effective diameter  16

39

EGA  3, 9, 24, 25, 26, 30

Element  16

Element currents  4, 22

Element diameter  21

Element lengths  26

Element material  19, 34

Element position  2, 34

Element Q  15

Element reactances  18

Element reinforcement  16

Element self-impedances  15

Element spacing  34, 35

Element Tapering  15

Element tip  19

Element-current distribution  19

Element-Current Profile  22, 27

Element-mounting brackets  16

Elements  32

Elements Menu  9, 13, 19

Elements with questionable tuning  22

Elevation  24

Elevation angle  4, 5

Elliptical plots  25

EME arrays  5

End  3, 22

Enter  2, 3, 9, 19, 22

Epson-compatible  30

Epson-compatible dot-matrix printers  26

Equivalent cylindrical elements  9

Equivalent diameter  16

Erroneous results  27

Error  2, 21, 31, 32

Error Beep  31

Esc  3, 4, 9, 19

Evolution of a design  29

Exit  4

Exit YO  2

Expand  10

Experiment  4, 7, 9, 14, 27, 28, 33

Experimenting with tubing diameters  18

F/B  5, 6, 27

F/B region  5, 23

F/B tradeoff  6

F10  22, 23, 26, 30

F5  25

F8  23

F9  22

Fascinating to ponder  22

Fascinating to watch  29

Fast computer  32

Fast CPU  29

Favor a certain portion of a band  11

Feed impedance  13

Feedline  11

Feedline impedance  11, 36

40

Ferromagnetic element material  20

Figure-of-merit  20

File format  18

File notes  2

File which already exists  2

Filename  2, 3, 23

Final dimensions  10

Final Yagi dimensions  2

First design  10

Fix/free  9

Fixed during automatic optimization  13

Fixed element dimensions  9

Flat mounting plates  16

Flat plate  17

Flatness parameter  7

Folded dipole  12

FOM  20

Forcing flat response  7

Forward gain  6, 13

Free  9

Free space  5, 26, 30

Free-space models  25

Free-space patterns  24

Frequencies  4, 34

Frequency  19

Frequency band  4, 13, 15, 18

Frequency bias  15

Frequency Graphs  23, 32

Frequency menu  5

Frequency offset  21, 22

Frequency range  23

Frequency response  17

Frequency Scaling  18

Frequency steps  23, 32

Frequency units  34

Frequency-scaling  19

Front-to-back  5

Function keys  31

G3SEK  17

Gain differences  30

Gain figure-of-merit  4, 20, 33

Gain figures  30

Gain FOM  21

Gain formula  33

Gain reference  30

Gain tradeoff  6

Gain/boomlength efficiency  21

Gamma Match  12

Generalized F/B  5

GHz  34

Global optimum  8

Gradient  8

GRAFTABL  3

Graph comparison  23

Graph resolution  32

41

Graph-comparison feature  10

GRAPHICS  3, 26

Graphics buffers  23, 24

Graphics card  29

Graphs  23, 32

Ground  24, 25, 34

Ground and Stacking  5

Ground menu  5

Groundplane  5

Guy wires  21

H-plane  6, 24, 26

Hairpin Match  11, 13

Ham Radio Magazine  12

Heavily tapered elements  15

Height  5, 17, 34

HERCULES  3

HF  11, 13, 15, 19, 21

HF elements  32

HF Yagis  6

HGC  3, 24, 26, 30

Hide the main menu  4, 9

High capacitance value  12

High-frequency rolloff region  21

High-resolution patterns  23

High-resolution plots  30, 32

Highest average gain and F/B  7

Highlighted by color  9

Highlighted file  3

Hill-climbing method  7

History of design changes  10

Hoch  17, 20

Home  3, 22

Horizontal polarization  24

Horizontally polarized Yagis  26

HP DeskJet  26

HP LaserJet  26

HP LaserJet or DeskJet printer  31

Hy-Gain  16

Hy-Gain element clamps  11

I key  9

IACS  20

Impedance  4, 7

Impedance curve  23

Impedance levels  22

Impedance tradeoff  7

Impractical antennas  21

Incrementing digit  24

Inductance  12

Innermost taper section  16, 19

Input Files  2

Input impedance  6

Input-impedance modeling error  13

Ins  9

Insulated through-the-boom mounting  18

42

Intermediate designs  10

International Annealed Copper Standard  20

Interrupt the optimizer  9

Isotropic antenna  30

Item last entered  4

Iteration  29

Iteration count  4, 29

Iteration step  8

Iteration step-size  8

J  4

Jaggard  16

Joint position  13

K1GQ  21, 33

Keywords  34

KHz  34

KLM  13, 14, 36

L-network  11

L4 norm  5

Landscape plot  31

Large mounting brackets  17

Last-director length  9

Lawson  15

LDEL  31

Lead inductance  12

Lead length  11, 12

Leeson  15

Length lines  35

Length rounding  10

Length unit  36

Lightbar  3

Limit  7

Limitations  10

Line-delete  22, 31

Linear-dB  23, 24, 32

Linear-regression curve-fit  20

Listing files  28

Local optimum  8

Log-dB scale  24

Logarithmic average  12

Long-boom designs  22

Long-boom VHF/UHF Yagis  1

Long-boom Yagis  20, 23

Losses  19, 26

Low impedances  21

Low-impedance Yagis  22

LPT  2, 31

M key  11

Main menu  3, 16

Match  4

Match frequency  11

Match menu  9, 11, 13

Matching Networks  11

43

Math coprocessor  1, 29

Mathematical model  1

Maximum forward gain  20

Maximum number of taper sections  34

Maximum-gain designs  22

Maximum-possible gain  20

Measurement unit  19

Method-of-moments  18

MHz  34

Middle frequency  27

Migrate closer  7, 26, 32

Minimizes worst-case interference  6

Minimum Element Spacing  32

Minimum SWR  11

MININEC  1, 15

MININEC calibration  21, 27

Minor lobe structure  24

MN  14

Model boom effects  17

Modeling environment  5

Modest computer  1

Modified algorithm  8

Modified-W2PV algorithm  15

Monotonic  27

Mounting bracket  16

Mounting brackets  21

Multiple peaks  8

Multiple plots  26

Mutual impedances  5

Myers  21

NEC  1

NEC calibration  21

Network dimensions  11

Network types  11

New filenames  24

New taper diameters  19

New taper lengths  19

New-element length  9

Nonadjustable element lengths  26

None  16

Noninsulated through-the-boom mounting  17

Normalize compared plots  24

Normalized plots  30

Notepad  22

Notepad Editor Keys  31

Notes  4, 22

Null at 90 degrees  26

Number of elements  34

Number of Frequency Steps for Graphs  32

Numerical Electromagnetics Code  1

Numerical input  4

NX1H  26

Ohmic losses  19

Ohms  4, 22

44

Omnidirectional noise  6

Optimal boomlength  10

Optimal dimensions  8

Optimization  32

Optimization Method  7

Optimization summing weights  6

Optimize joint driven-element position  13

Optimize just a few elements  9

Optimizer balks  32

Optimizing at three frequencies  7

Optimum SWR characteristics  14

Option letters  24

Optional lines  34

Options menu  2, 5, 6, 16, 19, 22, 26

Other  3

OUT.YAG  2, 28

Output filename  2

Output Files  2, 22, 27

Output-File Overwrite Check  32

Over-ground patterns  24

Overflow  5

Overlay  24, 26

Overwriting  2, 32

P command  23

Parameter tradeoffs  6

Parameters  27

Path  2, 29

Pattern  13, 23

Pattern Plots  23

Pattern-Quality Measure  5

Peak backlobe  6

Peak power  6

Percent  6

Perfectly matched  11

Permeability  19

Perturbations of antenna dimensions  8

PgDn  3, 22

PgUp  3, 22

Phasing line  13

Phasing-line impedance  36

Phasing-line tension  13

Phosphor Bronze  20

Physical Design of Yagi Antennas  15

Plot file  3, 23, 28

Plot file is overwritten  23

Plot Normalization  30

Plot Resolution  30

PLT  23

Polar coordinates  24

POLAR environment variable  24

Polarization  5

Position of a single driven element  9

PostScript  26

Power  5

Power-computation method  6

45

Preserving plots for later comparison  24

PRINT command  2

Print screen  25

Printed  2

Printer Port  31

Printer Type  30

Printing The Screen  26

PRN  2

Profile  22

Proximity of other antennas  13

PrtSc  22, 26

PSPLOT  26

Quick check  3

Quick reoptimization  18

Quirk  32

Radial scales  25

Real-world  21

Real-world performance  15

Rear horizon  6

Recall a previous design  10

Recall a saved graph  23

Recalled design  10

Rectangular coordinates  24

Redefine keys  22

Redo  10, 27

Reference dB  30

Reflector element  14

Region  5

Relative importance  6

Replicate a Yagi design  18

Resets the design history  10

Resistivity  19

Resolution  30

Resonance  21

Resonant frequency  17

Resonant length  11

Retapering  10

Return to the main menu  4

Ring  20

RMS power  5

Rods  11

Rounding  10

Running YO  3

Save  3, 4

Save color codes  26

Save Colors command  27

Save command  2, 10

Save Options  27

Save the current graph  23

SAVE.YAG  2

Scale  18, 19, 30

Scale markings  23

Scale menu  14

46

Screen  29

Screen Bounce  29

Screen clutter  4, 27

Screen Colors  25

Screen-update overhead  29

Screen-Update Rate  29

Self-impedance  15, 18

Self-impedance errors  15

Separators  34

SET BEEP  31

SET BOUNCE  29

SET DB  30

SET FCHECK  32

SET FOM  33

SET LDEL  31

SET LPT  31

SET MINSP  32

SET NFREQ  32

SET NORM  30

SET PINS  31

SET PLT  28

SET POLAR  32

SET RES  30

SET TRACE  32

SET UPDATE  29

SET WDEL  31

SET YAG  28

SHELL  28

Shortened driven element  11

Shorting strap  12, 14

Shrink the design  10

Shunt capacitance  11

Shunt inductance  11

Shunt-capacitance estimation error  13

Sidelobes  5

Signal-to-noise ratio  6

Silver  20

Single analysis frequency  4

Single-frequency optimization  7

Sketches  23, 32

Skin-effect  19, 22

Slow computer  1, 32

Snag  32

Source magnitudes and phases  14

Sources of error  21

Spacebar  4, 9, 23

Spacers  14

Spaces, commas, and tabs  34

Spacing between phasing straps  14

Spacing limit  32

Speed  1, 5

Spot-frequency  4, 18, 23, 27

Stack  10

Stacked pair of Yagis  35

Stacked Yagis  5

Stacking distance  5, 34, 35

47

Stainless steel  20

Starting configurations  9

Steel  20

Steepest-descent  7

Step size  8

Strategy for optimizing a dual-driven Yagi  14

Subdirectories  28

Subdirectory list  29

Submenu  3, 4

Switched plot comparison  24

SWR  6, 7, 14

SWR bandwidth  6

SWR tradeoff  7

Symbol  4, 19, 34, 36

T Match  12

T match without capacitors  12

Tabs  34

Taper conversion  15

Taper diameters  34

Taper lengths  34

Taper Modification  18

Taper schedule  15, 16, 18, 19, 34

Tapered AO files  27

Tapered elements  9

Tapered Yagi file  16

Tapering algorithm  15, 19, 27

Telescoping tubing  15

Terminate data entry  4

Terminating optimization  8

Terwilliger  26

Thick plot traces  32

Thicker elements  21

Three-frequency optimization  7

Time interval  29

Tip length  16, 18

Tip section  19

Title  22, 34

Tolles  12

Too Big  17

Total power  5

Trace thickness  24, 32

Tradeoff percentages  7

Tradeoffs  6

Tradeoffs menu  6, 13

Transformer Match  11

Transformer match frequency  34

Transmission line  11, 12

Trim existing elements for best performance  26

Truncates titles  34

Tubing swage  27

Tweezers  5

Two-Element Yagis  14

TYPE command  2

48

Underlining  9

Undesirable side-effects  6

Undo  10, 27

Undoing Changes  10

Uniform  7

Units  2, 27

Untapered elements  14, 27

Up-arrow  22

Updating the screen  29

Upper or lower case  34

Upper-case Z  9

Var  19

Velocity factor  13

Vertically polarized Yagis  26

Very short innermost taper sections  16

Very short taper sections  14

VGA  3, 9, 24, 25, 26, 30

VHF/UHF  19, 21, 22

Viewing graphs  23

Volume resistivity  20

W  4

W2PV  1, 15, 17, 18, 19

W6QHS  12, 15, 16, 19

W7ITB  12

WA2PHW  18, 20, 33

Wavelength units  4

WB0DGF  12

WDEL  31

Weighted combination  8

Weighted sum  6

White  17

Whole screen  4, 9

Whole-element length  33

Width  17

Wild-goose chases  8

Wind  32

Word-delete  22, 31

Wordprocessor  2

Worst-case SWR  7

Wraps words  22

X key  9

X-axis cutoff  24, 25

Y-axis cutoff  24, 25

YAG  2

Yagi Antenna Design  15

Yagi file  3

Yagi File Format  33

Yagi sketch  10, 13, 22

YO analysis kernel  15

YO PLOT  3, 24

YO-format output files  2

YO.INI  26, 27

Z  4

49