# RS232 DB25

| Pin No. | Name | Notes/Description | Pin No. | Name | Notes/Description |
| --- | --- | --- | --- | --- | --- |
| 1 | - | Protective/shielded ground | 14 | STD | Secondary Transmit Data |
| 2 | TD | Transmit Data (a.k.a TxD, Tx) | 15 | DB | Transmit Clock (a.k.a TCLK, TxCLK) |
| 3 | RD | Receive Data (a.k.a RxD, Rx) | 16 | SRD | Secondary Receive Data |
| 4 | RTS | Request To Send | 17 | DD | Receive Clock (a.k.a. RCLK) |
| 5 | CTS | Clear To Send | 18 | LL | Local Loopback |
| 6 | DSR | Data Set Ready | 19 | SRTS | Secondary Request to Send |
| 7 | SGND | Signal Ground | 20 | DTR | Data Terminal Ready |
| 8 | CD | Carrier Detect (a.k.a DCD) | 21 | RL/SQ | Signal Quality Detector/Remote loopback |
| 9 | - | Reserved for data set testing | 22 | RI | Ring Indicator (DCE raises when incoming call detected used for auto answer applications) |
| 10 | - | Reserved for data set testing | 23 | CH/CI | Signal Rate selector |
| 11 | - | Unassigned | 24 | DA | Auxiliary Clock (a.k.a. ACLK) |
| 12 | SDCD | Secondary Carrier Detect | 25 | - | Unassigned |
| 13 | SCTS | Secondary Clear to send | - | - | - |

# RS232 DB9

| Pin No. | Name | Notes/Description |
| --- | --- | --- |
| 1 | DCD | Data Carrier Detect |
| 2 | RD | Receive Data (a.k.a RxD, Rx) |
| 3 | TD | Transmit Data (a.k.a TxD, Tx) |
| 4 | DTR | Data Terminal Ready |
| 5 | SGND | Ground |
| 6 | DSR | Data Set Ready |
| 7 | RTS | Request To Send |
| 8 | CTS | Clear To Send |
| 9 | RI | Ring Indicator |